# region 库文件

import inspect

import os

import re

import signal

import sys

import time

import numpy as np

from brick_process_recording import BrickProcessRecorder

from ImageProcessing.BLX_Image_Camera import Blinx_Image_Camera

from ImageProcessing.Conversion_3D import Blinx_3D_Conversion

import cv2

from PyQt5 import QtGui, QtWidgets, QtCore

from PyQt5.QtCore import QRegExp, Qt, QEvent, QPropertyAnimation

from PyQt5.QtCore import QTimer

from PyQt5.QtGui import QRegExpValidator, QPixmap, QPainter, QColor

from PyQt5.QtWidgets import *

from Blinx_Jaka_Socket import *

from Blinx_Jaka_Rev_Socket import *

from Blinx_Object_Recongnition import *

from Blinx_Public_Class import *

from Blinx_Mech3D_Viewer_SDK import *

from ImageProcessing.image_recognition import *

from MainWindows import Ui_MainWindow

# endregion

# endregion

class Blinx_XXXY_Robot_Vision(QMainWindow, Ui_MainWindow, ):

# region 变量初始化

def __init__(self):  # 创建构造函数

super().__init__()  # 调用父类函数，继承

self.setupUi(self)  # 调用UI

self.graphics_view_init()  # graphics_view初始化

# 绑定空格键,按空格键保存图像

self._display_source_pixmaps = {}

self._init_recognition_display_widgets()

self.shortcut = QShortcut(Qt.Key_Space, self)

self.shortcut.activated.connect(self.on_space_pressed)

# 对象初始化

self.public_class = Blinx_Public_Class()

# yolo图像识别是初始

self.yolo_iamge = Blinx_image_rec(self.public_class)

# 标定转换3D

self.image_camera = Blinx_Image_Camera()   # 图像坐标转相机坐标

self.conversion_3d = Blinx_3D_Conversion()   # 标定转化

self.brick_process_recorder = None

# 创建 QTimer 实例

self.timer_ini = QTimer(self)  # 连接初始化定时器

# self.time_judge_client_state = QTimer(self)  # 通讯状态连接定时器

# 设置定时器触发的时间间隔

self.timer_ini.setInterval(5000)

# self.time_judge_client_state.setInterval(500)

# 连接定时器的 timeout() 信号到槽函数

self.timer_ini.timeout.connect(self.ini_class)

# self.time_judge_client_state.timeout.connect(self.judge_client_state)

# 设置定时器为单次触发

self.timer_ini.setSingleShot(True)  # 初始化一次

# 启动定时器

self.timer_ini.start()

# 机器人状态获取线程

self.timer_jaka_state = QTimer()

self.timer_jaka_state.setInterval(100)

self.timer_jaka_state.timeout.connect(self.get_jaka_state)

# Mech3D相机数据采集显示

self.timer_mech = QTimer()

self.timer_mech.setInterval(30)

self.timer_mech.timeout.connect(self.show_camera_mech)

# 流程程序定时器监控

self.timer_process = QTimer()

self.timer_process.setInterval(200)

self.timer_process.timeout.connect(self.blinx_timer_process)

self.timer_process.start()

# 标定是否ok

self.biaoding_ok = False

self.biaoding_image = None

self.btn_systermRun_click()  # 程序启动在系统运行界面

# 设置窗口大小

# 隐藏所有的Tab widget页面

self.tabBar = self.tabWidget.findChild(QTabBar)

self.tabBar.hide()

self.calibration_parm_ini()  # 标定参数初始化

self.pixel_point = []

# 槽函数

self.slot_init()  # 设置槽函数

self.joint_or_xyz = False

self.lineEdit_speed.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_x_1.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_y_1.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_x_2.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_y_2.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_x_3.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_y_3.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_x_4.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_P_y_4.setValidator(QRegExpValidator(QRegExp("[0-9]+$")))

self.lineEdit_R_x_1.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))  # 允许输入正负数和最多一位小数

self.lineEdit_R_y_1.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_R_x_2.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_R_y_2.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_R_x_3.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_R_y_3.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_R_x_4.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_R_y_4.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

self.lineEdit_absolute_loaction.setValidator(QRegExpValidator(QRegExp(r"^[-+]?[0-9]*\.?[0-9]+$")))

# 背景文字提示

self.lineEdit_absolute_loaction.setPlaceholderText("输入位置")

# endregion

# region 槽函数

def reset_rotation_history(self, process_name):

if process_name == "brick":

self.public_class.last_brick_rotation_delta = None

elif process_name == "ceramic":

self.public_class.last_ceramic_rotation_delta = None

def select_shortest_rotation_delta(self, process_name, angle_deg):

attr_name = f"last_{process_name}_rotation_delta"

previous_angle = getattr(self.public_class, attr_name, None)

return resolve_shortest_rotation_delta(previous_angle, angle_deg)

def remember_rotation_delta(self, process_name, angle_deg):

attr_name = f"last_{process_name}_rotation_delta"

setattr(self.public_class, attr_name, float(angle_deg))

def _normalize_pose_values(self, values):

if not isinstance(values, (list, tuple, np.ndarray)) or len(values) < 6:

return None

try:

return [float(values[index]) for index in range(6)]

except (TypeError, ValueError):

return None

def _summarize_pick_result(self, pick_result):

if not isinstance(pick_result, dict):

return None

summary = {}

fields = [

"source",

"class_id",

"score",

"pixel_x",

"pixel_y",

"depth_mm",

"angle_deg",

"raw_pca_angle_deg",

"valid_depth_count",

"valid_depth_ratio",

"angle_fallback",

"axis_ratio",

"is_valid",

"geometry_score",

"decision_status",

"decision_reason",

"decision_warning",

"rgb_low_quality",

"rgb_low_quality_reasons",

"rgb_depth_match_found",

"match_center_distance_px",

"match_angle_delta_deg",

"match_depth_delta_mm",

"match_bbox_iou",

"match_mask_iou",

"matched_depth_geometry_score",

"robot_x",

"robot_y",

]

for field in fields:

if field not in pick_result:

continue

value = pick_result[field]

if isinstance(value, np.generic):

value = value.item()

summary[field] = value

return summary

def _brick_record_snapshot(self):

return {

"brick_process_state": bool(self.public_class.brick_process_state),

"brick_process_node": self.public_class.brick_process_node,

"brick_process_num": int(self.public_class.brick_process_num),

"tcp_pose": self._normalize_pose_values(self.public_class.tcp_pos),

"joint_pos": self._normalize_pose_values(self.public_class.joint_pos),

"new_data": self._normalize_pose_values(self.public_class.new_data),

"brick_process_result": self._summarize_pick_result(self.public_class.brick_process_result),

"brick_secondary_alignment_result": self._summarize_pick_result(

getattr(self.public_class, "brick_secondary_alignment_result", None)

),

"last_brick_rotation_delta": getattr(self.public_class, "last_brick_rotation_delta", None),

}

def _start_brick_process_recording_session(self):

try:

if self.brick_process_recorder is not None:

self.brick_process_recorder.finalize_case(

status="interrupted_by_new_session",

process_node=self.public_class.brick_process_node,

public_snapshot=self._brick_record_snapshot(),

extra={"reason": "A new brick recording session replaced the previous one."},

)

session_name = time.strftime("brick_%Y%m%d_%H%M%S", time.localtime())

self.brick_process_recorder = BrickProcessRecorder(session_name=session_name)

session_message = f"墙砖记录目录: {self.brick_process_recorder.session_dir}"

print(session_message)

self.textEdit_log.append(session_message + "\n")

except Exception as exc:

self.brick_process_recorder = None

print("墙砖记录器初始化失败:", exc)

self.textEdit_log.append(f"墙砖记录器初始化失败: {exc}\n")

def _brick_record_start_case(self, raw_rgb, depth_map, display_rgb, extra=None):

if self.brick_process_recorder is None:

return

try:

self.brick_process_recorder.start_case(

process_num=self.public_class.brick_process_num,

process_node=self.public_class.brick_process_node,

raw_rgb=raw_rgb,

depth_map=depth_map,

display_rgb=display_rgb,

public_snapshot=self._brick_record_snapshot(),

extra=extra or {},

)

except Exception as exc:

print("墙砖记录器 start_case 失败:", exc)

def _brick_record_capture(self, capture_name, raw_rgb, depth_map, display_rgb, extra=None):

if self.brick_process_recorder is None:

return

try:

self.brick_process_recorder.record_capture(

capture_name=capture_name,

raw_rgb=raw_rgb,

depth_map=depth_map,

display_rgb=display_rgb,

process_node=self.public_class.brick_process_node,

public_snapshot=self._brick_record_snapshot(),

extra=extra or {},

)

except Exception as exc:

print("墙砖记录器 record_capture 失败:", exc)

def _brick_record_event(self, event_name, command_target=None, extra=None):

if self.brick_process_recorder is None:

return

try:

self.brick_process_recorder.record_event(

event_name=event_name,

process_node=self.public_class.brick_process_node,

public_snapshot=self._brick_record_snapshot(),

command_target=command_target,

extra=extra or {},

)

except Exception as exc:

print("墙砖记录器 record_event 失败:", exc)

def _brick_record_finalize_case(self, status, extra=None):

if self.brick_process_recorder is None:

return

try:

self.brick_process_recorder.finalize_case(

status=status,

process_node=self.public_class.brick_process_node,

public_snapshot=self._brick_record_snapshot(),

extra=extra or {},

)

except Exception as exc:

print("墙砖记录器 finalize_case 失败:", exc)

def _brick_move_linear_fast(self, data):

self.jaka.blinx_moveL(data, self.public_class.brick_linear_speed_fast, 5000, 0)

def _brick_move_linear_pick(self, data):

self.jaka.blinx_moveL(data, self.public_class.brick_linear_speed_pick, 5000, 0)

def _brick_joint_move_rotate(self, data):

self.jaka.blinx_joint_move(

0,

data,

self.public_class.brick_joint_speed_rotate,

50,

)

def _brick_slider_move_to(self, position):

self.jaka.blinx_set_analog_output(6, 26, self.public_class.brick_slider_speed)

self.jaka.blinx_set_analog_output(6, 25, position)

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 4, 1)

self.jaka.blinx_set_digital_output(6, 4, 0)

self.jaka.blinx_set_digital_output(6, 5, 0)

def _brick_restart_from_primary_pick(self, reason, extra=None):

restart_extra = {"reason": reason}

if isinstance(extra, dict):

restart_extra.update(extra)

self._brick_record_event(

"secondary_alignment_restart_to_primary",

extra=restart_extra,

)

self._brick_record_finalize_case(

"secondary_alignment_failed_restart",

extra=restart_extra,

)

self.public_class.brick_process_result = None

self.public_class.brick_secondary_alignment_result = None

self.public_class.new_data = None

self.reset_rotation_history("brick")

self.public_class.brick_process_node = "3-0"

def _init_recognition_display_widgets(self):

self._display_source_pixmaps = {

"Image_Show_1": None,

"Image_Show_2": None,

}

for label in (self.Image_Show_1, self.Image_Show_2):

label.setAlignment(Qt.AlignCenter)

label.setScaledContents(False)

self._update_recognition_display_layout()

def _update_recognition_display_layout(self):

main_rect = self.Image_Show_1.geometry()

if main_rect.width() <= 0 or main_rect.height() <= 0:

return

margin = max(12, int(min(main_rect.width(), main_rect.height()) * 0.02))

preview_width = max(320, int(main_rect.width() * 0.38))

preview_width = min(preview_width, int(main_rect.width() * 0.48))

preview_height = max(200, int(main_rect.height() * 0.40))

preview_height = min(preview_height, int(main_rect.height() * 0.48))

self.Image_Show_2.setGeometry(

main_rect.x() + margin,

main_rect.y() + margin,

preview_width,

preview_height,

)

self.Image_Show_2.raise_()

self._refresh_recognition_display_widgets()

def _render_display_pixmap(self, label, pixmap):

if pixmap is None or pixmap.isNull():

label.clear()

return

target_size = label.contentsRect().size()

if not target_size.isValid() or target_size.width() <= 0 or target_size.height() <= 0:

label.setPixmap(pixmap)

return

scaled_pixmap = pixmap.scaled(

target_size,

Qt.KeepAspectRatio,

Qt.SmoothTransformation,

)

label.setAlignment(Qt.AlignCenter)

label.setScaledContents(False)

label.setPixmap(scaled_pixmap)

def _set_display_pixmap(self, label, pixmap):

key = label.objectName()

if pixmap is None or pixmap.isNull():

self._display_source_pixmaps[key] = None

label.clear()

return

self._display_source_pixmaps[key] = QPixmap(pixmap)

self._render_display_pixmap(label, self._display_source_pixmaps[key])

def _set_display_image(self, label, image, image_format):

if image is None:

self._set_display_pixmap(label, None)

return

image_array = np.ascontiguousarray(image)

if image_array.ndim < 2:

self._set_display_pixmap(label, None)

return

height, width = image_array.shape[:2]

bytes_per_line = image_array.strides[0]

show_image = QtGui.QImage(

image_array.data,

width,

height,

bytes_per_line,

image_format,

).copy()

self._set_display_pixmap(label, QtGui.QPixmap.fromImage(show_image))

def _refresh_recognition_display_widgets(self):

for label_name in ("Image_Show_1", "Image_Show_2"):

pixmap = self._display_source_pixmaps.get(label_name)

if pixmap is not None:

self._render_display_pixmap(getattr(self, label_name), pixmap)

def showEvent(self, event):

super().showEvent(event)

QtCore.QTimer.singleShot(0, self._update_recognition_display_layout)

def resizeEvent(self, event):

super().resizeEvent(event)

self._update_recognition_display_layout()

def slot_init(self):

# 主界面按钮

self.btn_systermRun.clicked.connect(self.btn_systermRun_click)

self.btn_camSet.clicked.connect(self.btn_camSet_click)

self.btn_calibrate.clicked.connect(self.btn_calibrate_click)

# 末端执行器

self.btn_suction_get.clicked.connect(self.blinx_btn_suction_get)   # 吸盘取

self.btn_suction_set.clicked.connect(self.blinx_btn_suction_set)   # 吸盘放

self.btn_bundle_get.clicked.connect(self.blinx_btn_bundle_get)   # 捆扎机取

self.btn_bundle_set.clicked.connect(self.blinx_btn_bundle_set)   # 捆扎机放

# 流程启动按钮

self.btn_initialization.clicked.connect(self.blinx_btn_initialization)   # 初始化按钮

self.btn_porcelain.clicked.connect(self.blinx_btn_porcelain)   # 瓷砖贴片流程按钮

self.btn_brick.clicked.connect(self.blinx_btn_brick)   # 墙砖堆砌流程按钮

self.btn_rebar.clicked.connect(self.blinx_btn_rebar)   # 钢筋捆扎流程按钮

# TabPage2页面相机参数设置

self.bnOnceFrame.clicked.connect(self.get_one_image)

self.bnContinueFrame.clicked.connect(self.get_continue_frame)

self.bnSaveImage.clicked.connect(self.save_jpg)

# TabPage3页面相机参数设置

self.btn_trigger_biaoding.clicked.connect(self.get_image_mech)

self.bnSaveImage_biaoding.clicked.connect(self.save_jpg_biaoding)

# TabPage3标定参数设置

self.graphics_view_biaoding.mouseDoubleClickEvent = self.getPixel

self.btn_biaoding_star.clicked.connect(self.pixel_to_object)

self.btn_biaoding_save.clicked.connect(self.matrix_saveto_ini)

self.lab_R_X_1.mousePressEvent = self.lab_R_X_1_clicked

self.lab_R_Y_1.mousePressEvent = self.lab_R_Y_1_clicked

self.lab_R_X_2.mousePressEvent = self.lab_R_X_2_clicked

self.lab_R_Y_2.mousePressEvent = self.lab_R_Y_2_clicked

self.lab_R_X_3.mousePressEvent = self.lab_R_X_3_clicked

self.lab_R_Y_3.mousePressEvent = self.lab_R_Y_3_clicked

self.lab_R_X_4.mousePressEvent = self.lab_R_X_4_clicked

self.lab_R_Y_4.mousePressEvent = self.lab_R_Y_4_clicked

# TabPage3机器人控制器参数设置

self.btn_controller_power_on.clicked.connect(self.btn_controller_power_on_click)

self.btn_controller_power_off.clicked.connect(self.btn_controller_power_off_click)

self.btn_robot_power_on.clicked.connect(self.btn_robot_power_on_click)

self.btn_robot_power_off.clicked.connect(self.btn_robot_power_off_click)

self.btn_enable_robot.clicked.connect(self.btn_enable_robot_click)

self.btn_disable_robot.clicked.connect(self.btn_disable_robot_click)

self.btn_home.clicked.connect(self.btn_home_click)

self.btn_play.clicked.connect(self.btn_play_click)

# TabPage3机器人位置控制

self.btn_joint.clicked.connect(self.btn_joint_click)

self.btn_xyz.clicked.connect(self.btn_xyz_click)

# 关节1：加,减

self.btn_j1_add.clicked.connect(self.btn_j1_add_click)

self.btn_j1_subtract.clicked.connect(self.btn_j1_subtract_click)

# 关节2：加,减

self.btn_j2_add.clicked.connect(self.btn_j2_add_click)

self.btn_j2_subtract.clicked.connect(self.btn_j2_subtract_click)

# 关节3：加,减

self.btn_j3_add.clicked.connect(self.btn_j3_add_click)

self.btn_j3_subtract.clicked.connect(self.btn_j3_subtract_click)

# 关节4：加,减

self.btn_j4_add.clicked.connect(self.btn_j4_add_click)

self.btn_j4_subtract.clicked.connect(self.btn_j4_subtract_click)

# 关节5：加,减

self.btn_j5_add.clicked.connect(self.btn_j5_add_click)

self.btn_j5_subtract.clicked.connect(self.btn_j5_subtract_click)

# 关节6：加,减

self.btn_j6_add.clicked.connect(self.btn_j6_add_click)

self.btn_j6_subtract.clicked.connect(self.btn_j6_subtract_click)

# 速度按钮

self.btn_speed_min.clicked.connect(self.btn_speed_min_click)

self.btn_speed_max.clicked.connect(self.btn_speed_max_click)

# 步长按钮

self.btn_step_add.clicked.connect(self.btn_step_add_click)

self.btn_step_subtract.clicked.connect(self.btn_step_subtract_click)

# 扎丝机开/关

self.btn_zsj_open.clicked.connect(self.btn_zsj_open_click)

self.btn_zsj_close.clicked.connect(self.btn_zsj_close_click)

# 吸盘打开/关闭

self.btn_xipan_open.clicked.connect(self.btn_xipan_open_click)

self.btn_xipan_close.clicked.connect(self.btn_xipan_close_click)

# 快换打开/关闭

self.btn_kh_open.clicked.connect(self.btn_kh_open_click)

self.btn_kh_close.clicked.connect(self.btn_kh_close_click)

# 滑轨向左/向右

self.btn_servor_left.clicked.connect(self.btn_servor_left_click)

self.btn_servor_right.clicked.connect(self.btn_servor_right_click)

# 滑轨启动/停止/回原点

self.btn_servor_run.clicked.connect(self.btn_servor_run_click)

self.btn_servor_reset.clicked.connect(self.btn_servor_reset_click)

# 滑轨绝对速度

self.lineEdit_absolute_speed.textChanged.connect(self.absolute_speed_changed)

# 绝对位置

self.lineEdit_absolute_loaction.textChanged.connect(self.absolute_loaction_changed)

def graphics_view_init(self):

# 创建场景

self.scene_2d = QtWidgets.QGraphicsScene()

self.scene_depth = QtWidgets.QGraphicsScene()

self.scene_biaoding = QtWidgets.QGraphicsScene()

# 设置视图

self.graphics_view_2d.setScene(self.scene_2d)

self.graphics_view_depth.setScene(self.scene_depth)

self.graphics_view_biaoding.setScene(self.scene_biaoding)

# 启用抗锯齿

self.graphics_view_2d.setRenderHint(QtGui.QPainter.Antialiasing)

self.graphics_view_depth.setRenderHint(QtGui.QPainter.Antialiasing)

self.graphics_view_biaoding.setRenderHint(QtGui.QPainter.Antialiasing)

# 设置缩放控制

self.graphics_view_2d.setDragMode(QtWidgets.QGraphicsView.ScrollHandDrag)

self.graphics_view_depth.setDragMode(QtWidgets.QGraphicsView.ScrollHandDrag)

self.graphics_view_biaoding.setDragMode(QtWidgets.QGraphicsView.ScrollHandDrag)

# 设置缩放策略

self.graphics_view_2d.setTransformationAnchor(QtWidgets.QGraphicsView.AnchorUnderMouse)

self.graphics_view_depth.setTransformationAnchor(QtWidgets.QGraphicsView.AnchorUnderMouse)

self.graphics_view_biaoding.setTransformationAnchor(QtWidgets.QGraphicsView.AnchorUnderMouse)

# 在初始化时添加这些设置

self.graphics_view_2d.setInteractive(True)

self.graphics_view_depth.setInteractive(True)

self.graphics_view_biaoding.setInteractive(True)

# 设置缩放限制

self.graphics_view_2d.setMinimumSize(200, 200)

self.graphics_view_depth.setMinimumSize(200, 200)

self.graphics_view_biaoding.setMinimumSize(200, 200)

# 设置滚动条策略

self.graphics_view_2d.setHorizontalScrollBarPolicy(QtCore.Qt.ScrollBarAsNeeded)

self.graphics_view_2d.setVerticalScrollBarPolicy(QtCore.Qt.ScrollBarAsNeeded)

self.graphics_view_depth.setHorizontalScrollBarPolicy(QtCore.Qt.ScrollBarAsNeeded)

self.graphics_view_depth.setVerticalScrollBarPolicy(QtCore.Qt.ScrollBarAsNeeded)

self.graphics_view_biaoding.setHorizontalScrollBarPolicy(QtCore.Qt.ScrollBarAsNeeded)

self.graphics_view_biaoding.setVerticalScrollBarPolicy(QtCore.Qt.ScrollBarAsNeeded)

self.graphics_view_biaoding.viewport().setCursor(QtCore.Qt.ArrowCursor)  # 设置为普通鼠标箭头

# 禁用所有内置光标变更逻辑

self.graphics_view_biaoding.viewport().setAttribute(Qt.WA_SetCursor, False)  # 关键设置1

# 强制安装事件过滤器

self.graphics_view_biaoding.viewport().installEventFilter(self)

# 初始光标

self._current_cursor = Qt.ArrowCursor

# endregion

# region连接始化

def ini_class(self):

self.Object_recongnition = Blinx_Object_Recongnition(self.public_class)

self.jaka = Blinx_Jaka_Socket(self.public_class)

if self.jaka.jaka_connect_sucess == True:

self.label_jaka.setText("机器人连接成功")

self.label_jaka.setStyleSheet("color:rgb(0,200,0);border:none;")

self.textEdit_log.append("机器人连接成功！\n")

self.timer_jaka_state.start()

# self.jaka_rev=Blinx_Jaka_Rev_Socket(self.public_class)#机器人接受数据通道

else:

self.label_jaka.setText("机器人连接失败")

self.label_jaka.setStyleSheet("color:rgb(255,0,0);border:none;")

self.textEdit_log.append("Error：机器人连接失败\n")

self.mechCam = Blinx_Mech3D_Viewer_SDK(self.public_class)

self.mechCam.ConnectCamera()  # 连接相机

if self.public_class.mech_connected:

self.label_mech.setText("Mech3D相机连接成功")

self.label_mech.setStyleSheet("color:rgb(0,200,0);border:none;")

self.textEdit_log.append("Mech3D相机连接成功！\n")

self.timer_mech.start()

device = self.mechCam.camera_infos[self.mechCam.cam_index].model

devices = []

devices.append(device)

self.ComboDevices.addItems(devices)

else:

self.label_mech.setText("Mech3D相机连接失败")

self.label_mech.setStyleSheet("color:rgb(255,0,0);border:none;")

self.textEdit_log.append("Error：Mech3D相机连接失败\n")

# endregion

# 图像识别并经过标定转换为机器人XY坐标---Mech3D相机00

def image_robotic_coordinate_transformation_mechCam(self):

if self.public_class.mech_connected:

# 获取图像数据

self.public_class.mech_2d_image, self.public_class.mech_depth_map, self.public_class.mech_point_cloud = self.mechCam.GrabImages()

result_robot = []

if self.public_class.mech_2d_image is not None:

all_result_list, image_result = self.Object_recongnition.recognition(self.public_class.mech_img)

print("all_result_list_return:", all_result_list)

if len(str(all_result_list)) > 0:

for index, result in enumerate(all_result_list):

point_x, point_y = self.calibration(result[0][0], result[0][1])

result_robot.append((round(point_x, 3), round(point_y, 3), result[1], result[2], result[3]))

self.textEdit_log.append(

"Mech3D相机图像中心" + "_" + str(index) + ":" + str(result[0][0]) + "," + str(

result[0][1]) + "\n")

self.textEdit_log.append(

"机器人抓取坐标" + "_" + str(index) + ":" + str(round(point_x, 3)) + "," + str(

round(point_y, 3)) + "\n")

print("result_robot:", result_robot)

self.Image_Show_1.setPixmap(QPixmap())

showImage = QtGui.QImage(image_result.data, image_result.shape[1], image_result.shape[0],

QtGui.QImage.Format_RGB888)  # 设置显示RGB格式,opencv默认BGR

self.Image_Show_1.setPixmap(QtGui.QPixmap.fromImage(showImage))  # 显示图像

self.Image_Show_1.setScaledContents(True)  # 图像自适应窗口大小

self._set_display_image(self.Image_Show_1, image_result, QtGui.QImage.Format_RGB888)

return result_robot

# 图像识别并经过标定转换为机器人XY坐标

def image_robotic_coordinate_transformation(self, image):

if image is not None:

center, image_result = self.Object_recongnition.recognition(image)

if center[0] != 0 and center[1] != 0:

point_x, point_y = self.calibration(center[0], center[1])

self.textEdit_log.append("图像中心：" + str(center[0]) + "," + str(center[1]) + "\n")

self.textEdit_log.append("机器人抓取坐标：" + str(round(point_x, 3)) + "," + str(round(point_y, 3)) + "\n")

self.image_show_enable = False  # 显示使能关闭

self.public_class.mech_img = None

self.Image_Show_1.setPixmap(QPixmap())

showImage = QtGui.QImage(image_result.data, image_result.shape[1], image_result.shape[0],

QtGui.QImage.Format_BGR888)  # 设置显示BGR格式,opencv默认BGR

self.Image_Show_1.setPixmap(QtGui.QPixmap.fromImage(showImage))  # 显示图像

self.Image_Show_1.setScaledContents(True)  # 图像自适应窗口大小

self._set_display_image(self.Image_Show_1, image_result, QtGui.QImage.Format_BGR888)

self.image_show_enable = True  # 显示使能打开

return [round(point_x, 3), round(point_y, 3)]

else:

showImage = QtGui.QImage(image_result.data, image_result.shape[1], image_result.shape[0],

QtGui.QImage.Format_BGR888)  # 设置显示BGR格式,opencv默认BGR

self.Image_Show_1.setPixmap(QtGui.QPixmap.fromImage(showImage))  # 显示图像

self.Image_Show_1.setScaledContents(True)  # 图像自适应窗口大小

self._set_display_image(self.Image_Show_1, image_result, QtGui.QImage.Format_BGR888)

return [0, 0]

# endregion

# region Page2相机参数设置

def show_camera_mech(self):

try:

if self.public_class.mech_connected and self.public_class.is_continue_show:

# 获取图像数据

self.public_class.mech_2d_image, self.public_class.mech_depth_map, self.public_class.mech_point_cloud = self.mechCam.GrabImages()

# 处理并显示2D图像

self.process_and_show_2d_image()

# 处理并显示深度图像

self.process_and_show_depth_image()

except Exception as e:

print("show_camera_mech:", e)

def get_one_image(self):

self.public_class.is_continue_show = False

try:

if self.public_class.mech_connected:

# 获取图像数据

self.public_class.mech_2d_image, self.public_class.mech_depth_map, self.public_class.mech_point_cloud = self.mechCam.GrabImages()

# 处理并显示2D图像

self.process_and_show_2d_image()

# 处理并显示深度图像

self.process_and_show_depth_image()

except Exception as e:

print("show_camera_mech:", e)

def process_and_show_2d_image(self):

# 清除旧场景

self.scene_2d.clear()

# 转换图像格式

height, width = self.public_class.mech_2d_image.shape[:2]

bytes_per_line = 3 * width

q_img = QtGui.QImage(self.public_class.mech_2d_image.data, width, height,

bytes_per_line, QtGui.QImage.Format_RGB888)

# 创建QPixmap并添加到场景

pixmap = QtGui.QPixmap.fromImage(q_img)

pixmap_item = self.scene_2d.addPixmap(pixmap)

pixmap_item.setTransformationMode(QtCore.Qt.SmoothTransformation)

# 设置场景大小

self.scene_2d.setSceneRect(0, 0, width, height)

# 重置视图变换

self.graphics_view_2d.resetTransform()

# 新增：初始自适应视图大小

self.fit_2d_image_to_view()

def process_and_show_depth_image(self):

# 清除旧场景

self.scene_depth.clear()

# 处理深度图像

depth_8bit = cv2.normalize(self.public_class.mech_depth_map, None, 0, 255,

cv2.NORM_MINMAX, cv2.CV_8UC1)

depth_color = cv2.applyColorMap(depth_8bit, cv2.COLORMAP_JET)

depth_rgb = cv2.cvtColor(depth_color, cv2.COLOR_BGR2RGB)

# 转换图像格式

height, width = depth_rgb.shape[:2]

bytes_per_line = 3 * width

q_img = QtGui.QImage(depth_rgb.data, width, height,

bytes_per_line, QtGui.QImage.Format_RGB888)

# 创建QPixmap并添加到场景

pixmap = QtGui.QPixmap.fromImage(q_img)

pixmap_item = self.scene_depth.addPixmap(pixmap)

pixmap_item.setTransformationMode(QtCore.Qt.SmoothTransformation)

# 设置场景大小

self.scene_depth.setSceneRect(0, 0, width, height)

# 重置视图变换

self.graphics_view_depth.resetTransform()

# 新增：初始自适应视图大小

self.fit_depth_image_to_view()

# 新增方法：自适应2D图像到视图

def fit_2d_image_to_view(self):

if not self.scene_2d.items():

return

# 获取视图和场景的尺寸

view_rect = self.graphics_view_2d.viewport().rect()

scene_rect = self.scene_2d.sceneRect()

# 计算合适的缩放比例

x_ratio = view_rect.width() / scene_rect.width()

y_ratio = view_rect.height() / scene_rect.height()

ratio = min(x_ratio, y_ratio)

# 应用缩放

self.graphics_view_2d.resetTransform()

self.graphics_view_2d.scale(ratio, ratio)

# 确保图像居中

self.center_2d_image()

# 新增方法：自适应深度图像到视图

def fit_depth_image_to_view(self):

if not self.scene_depth.items():

return

# 获取视图和场景的尺寸

view_rect = self.graphics_view_depth.viewport().rect()

scene_rect = self.scene_depth.sceneRect()

# 计算合适的缩放比例

x_ratio = view_rect.width() / scene_rect.width()

y_ratio = view_rect.height() / scene_rect.height()

ratio = min(x_ratio, y_ratio)

# 应用缩放

self.graphics_view_depth.resetTransform()

self.graphics_view_depth.scale(ratio, ratio)

# 确保图像居中

self.center_depth_image()

# 新增方法：居中2D图像

def center_2d_image(self):

if not self.scene_2d.items():

return

self.graphics_view_2d.setSceneRect(self.scene_2d.sceneRect())

self.graphics_view_2d.centerOn(self.scene_2d.items()[0])

# 新增方法：居中深度图像

def center_depth_image(self):

if not self.scene_depth.items():

return

self.graphics_view_depth.setSceneRect(self.scene_depth.sceneRect())

self.graphics_view_depth.centerOn(self.scene_depth.items()[0])

def wheelEvent(self, event):

# 检查鼠标在哪个视图上

if self.graphics_view_2d.underMouse():

view = self.graphics_view_2d

elif self.graphics_view_depth.underMouse():

view = self.graphics_view_depth

elif self.graphics_view_biaoding.underMouse():

view = self.graphics_view_biaoding

else:

return

# 计算缩放因子

factor = 1.1 if event.angleDelta().y() > 0 else 0.9

# 应用缩放

view.scale(factor, factor)

def eventFilter(self, obj, event):

# 拦截所有可能改变光标的事件

if event.type() in [QEvent.MouseButtonPress,

QEvent.MouseButtonRelease,

QEvent.MouseMove,

QEvent.HoverEnter,

QEvent.HoverLeave]:

obj.setCursor(self._current_cursor)

# 额外拦截 QGraphicsView 内部事件

if event.type() == QEvent.GraphicsSceneMouseMove:

self.graphics_view_biaoding.viewport().setCursor(self._current_cursor)

return super().eventFilter(obj, event)

def mousePressEvent(self, event):

if event.button() == Qt.LeftButton:

# 拖拽开始时强制锁定

self._current_cursor = Qt.ArrowCursor

self.graphics_view_biaoding.viewport().setCursor(Qt.ArrowCursor)

self.graphics_view_biaoding.setDragMode(QGraphicsView.ScrollHandDrag)

super().mousePressEvent(event)

def mouseMoveEvent(self, event):

# 实时矫正（每秒60次检查）

if self.graphics_view_biaoding.viewport().cursor().shape() != self._current_cursor:

self.graphics_view_biaoding.viewport().setCursor(self._current_cursor)

super().mouseMoveEvent(event)

def mouseReleaseEvent(self, event):

# 拖拽结束后保持箭头

self._current_cursor = Qt.ArrowCursor

self.graphics_view_biaoding.viewport().setCursor(Qt.ArrowCursor)

super().mouseReleaseEvent(event)

def keyPressEvent(self, event):

if event.key() == Qt.Key_Control:

# 物理锁定滚动条

self.graphics_view_2d.verticalScrollBar().setEnabled(False)

self.graphics_view_2d.horizontalScrollBar().setEnabled(False)

self.graphics_view_depth.verticalScrollBar().setEnabled(False)

self.graphics_view_depth.horizontalScrollBar().setEnabled(False)

self.graphics_view_biaoding.verticalScrollBar().setEnabled(False)

self.graphics_view_biaoding.horizontalScrollBar().setEnabled(False)

super().keyPressEvent(event)

def keyReleaseEvent(self, event):

if event.key() == Qt.Key_Control:

# 恢复滚动条但不显示

self.graphics_view_2d.verticalScrollBar().setEnabled(True)

self.graphics_view_2d.horizontalScrollBar().setEnabled(True)

self.graphics_view_depth.verticalScrollBar().setEnabled(True)

self.graphics_view_depth.horizontalScrollBar().setEnabled(True)

self.graphics_view_biaoding.verticalScrollBar().setEnabled(True)

self.graphics_view_biaoding.horizontalScrollBar().setEnabled(True)

super().keyReleaseEvent(event)

def get_continue_frame(self):

self.public_class.is_continue_show = True

# ch:存图 | en:save image

def save_jpg(self):

now_time = time.strftime('%Y-%m-%d-%H-%M-%S', time.localtime(time.time()))  # 获取系统时间

if self.public_class.mech_2d_image is None:

strError = "Save jpg failed ret"

QMessageBox.warning(mainWindow, "Error", strError, QMessageBox.Ok)

else:

cv2.imwrite('pic/cam1/image_' + str(now_time) + '.jpg', self.public_class.mech_2d_image)  # 保存2D图片

cv2.imwrite('pic/cam1/image_' + str(now_time) + '.tiff', self.public_class.mech_depth_map)  # 保存深度图片

self.show_auto_close_message("图片已保存！")

print("Save image_1 success")

#按空格保存图像

def on_space_pressed(self):

if self.tabWidget.currentIndex() == 1:

if self.public_class.mech_2d_image is not None:

self.save_jpg()

def show_auto_close_message(self,message):

self.msg = QMessageBox(self)

self.msg.setWindowTitle("提示")

self.msg.setText(str(message))

self.msg.show()

# 定时关闭消息框

QTimer.singleShot(100, self.msg.close)

# endregion

# region 流程定时器逻辑

def blinx_timer_process(self):

# 判断是否是吸盘取放功能

if self.public_class.sucker_state:

# 判断吸盘是取还是放

if self.public_class.sucker_type == 0:   # 如果类别是取

# 判断执行器放置区域

if self.public_class.sucker_process == "0-0":

# 获取机械臂控制器的数字输入信号

self.jaka.blinx_get_digital_input_status()

time.sleep(0.5)

# 将吸盘捆扎机的放置区的状态提出

xp_state = self.robot_DI_1

kzj_state = self.robot_DI_2

# 判断吸盘与捆扎机是否都在

if xp_state == 1 and kzj_state == 1:

self.public_class.sucker_process = "1-0"

# 判断如果吸盘在捆扎机不在

elif xp_state == 1 and kzj_state != 1:

print("提示捆扎机在末端的提示语")

# 判断如果捆扎机在，吸盘不在

elif xp_state != 1 and kzj_state == 1:

print("提示吸盘已在末端上")

# 如果两个都不在

elif xp_state != 1 and kzj_state != 1:

print("报警，请将末端执行器归位")

self.public_class.sucker_state = False

# 控制机械臂旋转180度

elif self.public_class.sucker_process == "1-0":

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.sucker_process = "1-2"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.sucker_process == "1-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂到达吸盘上方

data = [float(self.public_class.sucker_actuator_loc[0]),

float(self.public_class.sucker_actuator_loc[1]),

float(self.public_class.sucker_actuator_loc[2]) + 30.00,

float(self.public_class.sucker_actuator_loc[3]),

float(self.public_class.sucker_actuator_loc[4]),

float(self.public_class.sucker_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-3"

# 判断坐标是否到达，如果到达先打开快换夹具，在控制机械臂到达吸盘位置

elif self.public_class.sucker_process == "1-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达吸盘位置

data = [float(self.public_class.sucker_actuator_loc[0]),

float(self.public_class.sucker_actuator_loc[1]),

float(self.public_class.sucker_actuator_loc[2]),

float(self.public_class.sucker_actuator_loc[3]),

float(self.public_class.sucker_actuator_loc[4]),

float(self.public_class.sucker_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-4"

# 判断坐标是否到达，如果到达控制快换夹具关，并控制机械臂上升

elif self.public_class.sucker_process == "1-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 0)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 10.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-5"

# 判断坐标是否到达，如果到达控制Y轴移动到

elif self.public_class.sucker_process == "1-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出吸盘放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1] + 100.00),

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-6"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.sucker_process == "1-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00, float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.sucker_process = "1-7"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.sucker_process == "1-7":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.sucker_process = "1-8"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.sucker_process == "1-8":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (

-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (

-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = None

self.public_class.sucker_process = "0-0"

self.public_class.sucker_state = False

else:

# 判断执行器放置区域

if self.public_class.sucker_process == "0-0":

# 获取机械臂控制器的数字输入信号

self.jaka.blinx_get_digital_input_status()

time.sleep(0.5)

# 将吸盘捆扎机的放置区的状态提出

xp_state = self.robot_DI_1

kzj_state = self.robot_DI_2

# 判断吸盘与捆扎机是否都在

if xp_state == 1 and kzj_state == 1:

print("吸盘已在放置区")

# 判断如果吸盘在捆扎机不在

elif xp_state == 1 and kzj_state != 1:

print("提示目前末端是捆扎机")

# 判断如果捆扎机在，吸盘不在

elif xp_state != 1 and kzj_state == 1:

self.public_class.sucker_process = "1-0"

# 如果两个都不在

elif xp_state != 1 and kzj_state != 1:

print("报警，请将末端执行器归位")

self.public_class.sucker_state = False

# 控制机械臂旋转180度

elif self.public_class.sucker_process == "1-0":

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.sucker_process = "1-2"

# 角度是否到达，如果到达控制机械臂待放置位置

elif self.public_class.sucker_process == "1-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂待放置位置

data = [float(self.public_class.sucker_actuator_loc[0]),

float(self.public_class.sucker_actuator_loc[1]) + 100.00,

float(self.public_class.sucker_actuator_loc[2]) + 10.00,

float(self.public_class.sucker_actuator_loc[3]),

float(self.public_class.sucker_actuator_loc[4]),

float(self.public_class.sucker_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-3"

# 判断坐标是否到达，如果到达先控制机械臂到达吸盘放置区位置上一公分位置

elif self.public_class.sucker_process == "1-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]) - 100.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-4"

# 判断坐标是否到达，并控制机械臂下降，再释放快换夹具

elif self.public_class.sucker_process == "1-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 10.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-5"

time.sleep(1)

# 快换夹具释放

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 判断坐标是否到达，如果到达控制z轴上升

elif self.public_class.sucker_process == "1-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出吸盘放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 30.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.sucker_process = "1-6"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.sucker_process == "1-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.sucker_process = "1-7"

# 角度是否到达，如果到达控制机械臂回到初始角度

elif self.public_class.sucker_process == "1-7":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.sucker_process = "1-8"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.sucker_process == "1-8":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (

-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (

-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = None

self.public_class.sucker_process = "0-0"

self.public_class.sucker_state = False

# 判断是否是捆扎机取放功能

elif self.public_class.bundle_state:

# 判断捆扎机是取还是放

if self.public_class.bundle_type == 0:   # 如果类别是取

# 判断执行器放置区域

if self.public_class.bundle_process == "0-0":

# 获取机械臂控制器的数字输入信号

self.jaka.blinx_get_digital_input_status()

time.sleep(0.5)

# 将吸盘捆扎机的放置区的状态提出

xp_state = self.robot_DI_1

kzj_state = self.robot_DI_2

# 判断吸盘与捆扎机是否都在

if xp_state == 1 and kzj_state == 1:

self.public_class.bundle_process = "1-0"

# 判断如果吸盘在捆扎机不在

elif xp_state == 1 and kzj_state != 1:

print("提示捆扎机在末端的提示语")

self.public_class.bundle_state = False

# 判断如果捆扎机在，吸盘不在

elif xp_state != 1 and kzj_state == 1:

print("提示吸盘在末端上")

self.public_class.bundle_state = False

# 如果两个都不在

elif xp_state != 1 and kzj_state != 1:

print("报警，请将末端执行器归位")

self.public_class.bundle_state = False

# 控制机械臂旋转180度

elif self.public_class.bundle_process == "1-0":

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-1"

# 角度是否到达，如果到达控制J6旋转90都

elif self.public_class.bundle_process == "1-1":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制末端移动90度

data = [float(self.public_class.joint_pos[0]), float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]),

float(self.public_class.joint_pos[5]) - 90.00]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-2"

# 角度是否到达，如果到达控制机械臂到达捆扎机上方

elif self.public_class.bundle_process == "1-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂到达捆扎机上方

data = [float(self.public_class.bundle_actuator_loc[0]),

float(self.public_class.bundle_actuator_loc[1]),

float(self.public_class.bundle_actuator_loc[2]) + 30.00,

float(self.public_class.bundle_actuator_loc[3]),

float(self.public_class.bundle_actuator_loc[4]),

float(self.public_class.bundle_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-3"

# 判断坐标是否到达，如果到达先打开快换夹具，在控制机械臂到达捆扎机位置

elif self.public_class.bundle_process == "1-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达捆扎机位置

data = [float(self.public_class.bundle_actuator_loc[0]),

float(self.public_class.bundle_actuator_loc[1]),

float(self.public_class.bundle_actuator_loc[2]),

float(self.public_class.bundle_actuator_loc[3]),

float(self.public_class.bundle_actuator_loc[4]),

float(self.public_class.bundle_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-4"

# 判断坐标是否到达，如果到达控制快换夹具关，并控制机械臂上升

elif self.public_class.bundle_process == "1-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 0)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 10.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-5"

# 判断坐标是否到达，如果到达控制Y轴移动到

elif self.public_class.bundle_process == "1-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出吸盘放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1] + 100.00),

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-6"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.bundle_process == "1-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-7"

# 角度是否到达，如果到达控制机械臂到达捆扎机过度位置

elif self.public_class.bundle_process == "1-7":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂捆扎机过度位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1] + 50.00),

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-8"

# 判断坐标是否到达，如果到达控制机械臂旋转180度

elif self.public_class.bundle_process == "1-8":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.joint_pos[0]) + 180.00,

float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]),

float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]),

float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-9"

# 角度是否到达，如果到达控制机械臂到达初始位置

elif self.public_class.bundle_process == "1-9":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (

-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (

-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = None

self.public_class.bundle_process = "0-0"

self.public_class.bundle_state = False

else:

# 判断执行器放置区域

if self.public_class.bundle_process == "0-0":

# 获取机械臂控制器的数字输入信号

self.jaka.blinx_get_digital_input_status()

time.sleep(0.5)

# 将吸盘捆扎机的放置区的状态提出

xp_state = 1  # self.robot_DI_1

kzj_state = 0  # self.robot_DI_2

# 判断吸盘与捆扎机是否都在

if xp_state == 1 and kzj_state == 1:

print("捆扎机已在放置区")

self.public_class.sucker_state = False

# 判断如果吸盘在捆扎机不在

elif xp_state == 1 and kzj_state != 1:

self.public_class.bundle_process = "1-0"

# 判断如果捆扎机在，吸盘不在

elif xp_state != 1 and kzj_state == 1:

print("提示目前末端是吸盘")

self.public_class.sucker_state = False

# 如果两个都不在

elif xp_state != 1 and kzj_state != 1:

print("报警，请将末端执行器归位")

self.public_class.sucker_state = False

# 控制机械臂到达过度点位

elif self.public_class.bundle_process == "1-0":

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]) - 50.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-1"

# 控制机械臂旋转180度

elif self.public_class.bundle_process == "1-1":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-2"

# 角度是否到达，如果到达控制机械臂待放置位置

elif self.public_class.bundle_process == "1-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂待放置位置

data = [float(self.public_class.bundle_actuator_loc[0]),

float(self.public_class.bundle_actuator_loc[1]) + 100.00,

float(self.public_class.bundle_actuator_loc[2]) + 10.00,

float(self.public_class.bundle_actuator_loc[3]),

float(self.public_class.bundle_actuator_loc[4]),

float(self.public_class.bundle_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-3"

# 判断坐标是否到达，如果到达先控制机械臂到达捆扎机放置区上方一公分位置

elif self.public_class.bundle_process == "1-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达捆扎机位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]) - 100.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-5"

# 判断坐标是否到达，并控制机械臂下降，再释放快换夹具

elif self.public_class.bundle_process == "1-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达捆扎机位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 10.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-6"

time.sleep(1)

# 快换夹具释放

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 判断坐标是否到达，如果到达控制z轴上升

elif self.public_class.bundle_process == "1-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出捆扎机器放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 30.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.bundle_process = "1-7"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.bundle_process == "1-7":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-8"

# 角度是否到达，如果到达控制机械臂回到初始角度

elif self.public_class.bundle_process == "1-8":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.bundle_process = "1-9"

# 角度是否到达，如果到达结束流程

elif self.public_class.bundle_process == "1-9":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (

-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (

-0.1 <= error_j6 <= 0.1):

self.public_class.new_data = None

self.public_class.bundle_process = "0-0"

self.public_class.bundle_state = False

# 判断是否是瓷砖贴片流程

elif self.public_class.ceramic_process_state:

# 判断执行器放置区域

if self.public_class.ceramic_process_node == "0-0":

# 获取机械臂控制器的数字输入信号

self.jaka.blinx_get_digital_input_status()

time.sleep(0.5)

# 将吸盘捆扎机的放置区的状态提出

xp_state = self.robot_DI_1

kzj_state = self.robot_DI_2

# 判断吸盘与捆扎机是否都在

if xp_state == 1 and kzj_state == 1:

self.public_class.ceramic_process_node = "1-0"  # 如果两个末端执行器都x2在

# 判断如果吸盘在捆扎机不在

elif xp_state == 1 and kzj_state != 1:

self.public_class.ceramic_process_node = "2-0"  # 如果是吸盘就直接开始下一步

# 判断如果捆扎机在，吸盘不在

elif xp_state != 1 and kzj_state == 1:

self.public_class.ceramic_process_node = "3-0"  # 需先放置捆扎机，在获取吸盘

# 如果两个都不在

elif xp_state != 1 and kzj_state != 1:

print("报警，请将末端执行器归位")

self.public_class.ceramic_process_node = False

# 控制机械臂旋转180度

elif self.public_class.ceramic_process_node == "1-0":

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-1"

# 角度是否到达，如果到达控制J6旋转90都

elif self.public_class.ceramic_process_node == "1-1":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制末端移动90度

data = [float(self.public_class.joint_pos[0]), float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5]) + 90.00]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-2"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.ceramic_process_node == "1-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂到达吸盘上方

data = [float(self.public_class.sucker_actuator_loc[0]),

float(self.public_class.sucker_actuator_loc[1]),

float(self.public_class.sucker_actuator_loc[2]) + 30.00,

float(self.public_class.sucker_actuator_loc[3]),

float(self.public_class.sucker_actuator_loc[4]),

float(self.public_class.sucker_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-3"

# 判断坐标是否到达，如果到达先打开快换夹具，在控制机械臂到达吸盘位置

elif self.public_class.ceramic_process_node == "1-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达吸盘位置

data = [float(self.public_class.sucker_actuator_loc[0]),

float(self.public_class.sucker_actuator_loc[1]),

float(self.public_class.sucker_actuator_loc[2]),

float(self.public_class.sucker_actuator_loc[3]),

float(self.public_class.sucker_actuator_loc[4]),

float(self.public_class.sucker_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-4"

# 判断坐标是否到达，如果到达控制快换夹具关，并控制机械臂上升

elif self.public_class.ceramic_process_node == "1-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 0)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 10.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-5"

# 判断坐标是否到达，如果到达控制Y轴移动到

elif self.public_class.ceramic_process_node == "1-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出吸盘放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1] + 100.00),

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-6"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.ceramic_process_node == "1-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-7"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.ceramic_process_node == "1-7":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "1-8"

# 角度是否到达，如果到达控制机械臂到达吸盘上方

elif self.public_class.ceramic_process_node == "1-8":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (

-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (

-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "3-0"

# 控制机械臂到达过度点位

elif self.public_class.ceramic_process_node == "2-0":

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]) - 50.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "2-1"

# 控制机械臂旋转180度

elif self.public_class.ceramic_process_node == "2-1":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "2-2"

# 角度是否到达，如果到达控制机械臂待放置位置

elif self.public_class.ceramic_process_node == "2-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂待放置位置

data = [float(self.public_class.bundle_actuator_loc[0]),

float(self.public_class.bundle_actuator_loc[1]) + 100.00,

float(self.public_class.bundle_actuator_loc[2]) + 10.00,

float(self.public_class.bundle_actuator_loc[3]),

float(self.public_class.bundle_actuator_loc[4]),

float(self.public_class.bundle_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "2-3"

# 判断坐标是否到达，如果到达先控制机械臂到达捆扎机放置区上方一公分位置

elif self.public_class.ceramic_process_node == "2-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达捆扎机位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]) - 100.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.ceramic_process_node = "2-4"

# 判断坐标是否到达，并控制机械臂下降，再释放快换夹具

elif self.public_class.ceramic_process_node == "2-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

self.public_class.new_data = self._normalize_pose_values(self.public_class.tcp_pos)

self.public_class.brick_process_node = "3-7-6"

# 控制Z轴下降

elif self.public_class.brick_process_node == "3-7-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 70.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self._brick_record_event(

"secondary_descend_command",

command_target=data,

)

self._brick_move_linear_pick(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-7-7"

# 打开吸盘，控制Z轴上升

elif self.public_class.brick_process_node == "3-7-7":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 打开吸盘

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 6, 1)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) + 71.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self._brick_record_event(

"secondary_suction_and_lift_command",

command_target=data,

extra={

"suction_state": "open",

"executed_secondary_pick_tcp_pose": self._normalize_pose_values(self.public_class.tcp_pos),

},

)

self._brick_move_linear_pick(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-8"

# # 回到待放点位

# elif self.public_class.brick_process_node == "3-7-8":

#     error_x = (float(self.public_class.new_data[0]) - float(

#         self.public_class.tcp_pos[0]))

#     error_y = (float(self.public_class.new_data[1]) - float(

#         self.public_class.tcp_pos[1]))

#     error_z = (float(self.public_class.new_data[2]) - float(

#         self.public_class.tcp_pos[2]))

#     if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

#         # 控制机械臂到达放置过度点

#         data = [float(self.public_class.brick_excessive_loc[0]),

#                 float(self.public_class.brick_excessive_loc[1]),

#                 float(self.public_class.brick_excessive_loc[2]),

#                 float(self.public_class.brick_excessive_loc[3]),

#                 float(self.public_class.brick_excessive_loc[4]),

#                 float(self.public_class.brick_excessive_loc[5])]

#         self.jaka.blinx_moveL(data, 250, 5000, 0)

#         self.public_class.new_data = data

#         self.public_class.brick_process_node = "3-8"

# 判断机械臂是否达到位置，判断这是第几次放置，根据放置位置，进行放置

elif self.public_class.brick_process_node == "3-8":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 根据抓取次数进行放置

data = []

if self.public_class.brick_process_num == 0:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]),

float(self.public_class.brick_place_loc[1]),

float(self.public_class.brick_place_loc[2]) + 40.00,

float(self.public_class.brick_place_loc[3]),

float(self.public_class.brick_place_loc[4]),

float(self.public_class.brick_place_loc[5])]

elif self.public_class.brick_process_num == 1:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]) + 137.00,

float(self.public_class.brick_place_loc[1]),

float(self.public_class.brick_place_loc[2]) + 40.00,

float(self.public_class.brick_place_loc[3]),

float(self.public_class.brick_place_loc[4]),

float(self.public_class.brick_place_loc[5])]

elif self.public_class.brick_process_num == 2:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]) + 70.00,

float(self.public_class.brick_place_loc[1]) - 103.00,

float(self.public_class.brick_place_loc[2]) + 40.00,

float(179.694),

float(0.567),

float(46.242)]

elif self.public_class.brick_process_num == 3:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]) + 172.00,

float(self.public_class.brick_place_loc[1]) - 103.00,

float(self.public_class.brick_place_loc[2]) + 40.00,

float(self.public_class.brick_place_loc[3]),

float(self.public_class.brick_place_loc[4]),

float(self.public_class.brick_place_loc[5])]

elif self.public_class.brick_process_num == 4:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]) + 137.00,

float(self.public_class.brick_place_loc[1]) - 204.00,

float(self.public_class.brick_place_loc[2]) + 40.00,

float(self.public_class.brick_place_loc[3]),

float(self.public_class.brick_place_loc[4]),

float(self.public_class.brick_place_loc[5])]

elif self.public_class.brick_process_num == 5:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]),

float(self.public_class.brick_place_loc[1]) - 204.00,

float(self.public_class.brick_place_loc[2]) + 40.00,

float(self.public_class.brick_place_loc[3]),

float(self.public_class.brick_place_loc[4]),

float(self.public_class.brick_place_loc[5])]

elif self.public_class.brick_process_num == 6:

# 控制机械臂到达放置过度点

data = [float(self.public_class.brick_place_loc[0]) - 35.00,

float(self.public_class.brick_place_loc[1]) - 103.00,

float(self.public_class.brick_place_loc[2]) + 40.00,

float(179.694),

float(0.567),

float(46.242)]

if len(data) > 0:

self._brick_record_event(

"place_approach_command",

command_target=data,

extra={"brick_process_num": int(self.public_class.brick_process_num)},

)

self._brick_move_linear_fast(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-9"

else:

print("DATA 无数据")

# 判断机械臂是否达到位置，如果到达，下降Z轴

elif self.public_class.brick_process_node == "3-9":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达放置过度点

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 40.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self._brick_record_event(

"place_descend_command",

command_target=data,

)

self._brick_move_linear_pick(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-10"

# 判断机械臂是否达到位置，如果到达，关闭吸盘，Z轴上升

elif self.public_class.brick_process_node == "3-10":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 关闭吸盘

self.btn_xipan_close_click()

# 控制机械臂Z轴上升

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) + 50.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self._brick_record_event(

"place_release_and_lift_command",

command_target=data,

extra={"suction_state": "closed"},

)

self._brick_move_linear_pick(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-11"

# 判断机械臂是否达到位置，如果到达，回到转180位置

elif self.public_class.brick_process_node == "3-11":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制末端进行旋转

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self._brick_joint_move_rotate(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-12"

# 角度是否到达，如果到达再回到初始姿态

elif self.public_class.brick_process_node == "3-12":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制末端进行旋转

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self._brick_joint_move_rotate(data)

self.public_class.new_data = data

self.public_class.brick_process_node = "3-13"

#  角度是否到达，如果到达再回到初始姿态

elif self.public_class.brick_process_node == "3-13":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

if self.public_class.brick_process_num <= 5:

completed_num = int(self.public_class.brick_process_num)

self._brick_record_finalize_case(

"cycle_complete",

extra={

"completed_brick_process_num": completed_num,

"next_brick_process_num": completed_num + 1,

},

)

self.public_class.brick_process_num = self.public_class.brick_process_num + 1

self.public_class.brick_process_node = "3-1"

else:

# 回到初始位置

data = [float(self.public_class.identify_loc1[0]), float(self.public_class.identify_loc1[1]),

float(self.public_class.identify_loc1[2]), float(self.public_class.identify_loc1[3]),

float(self.public_class.identify_loc1[4]), float(self.public_class.identify_loc1[5])]

self._brick_move_linear_fast(data)

self._brick_record_finalize_case(

"brick_process_complete",

extra={"return_target": data},

)

self.public_class.new_data = None

self.public_class.brick_process_state = False  # 墙砖流程状态

self.public_class.brick_process_node = "0-0"  # 墙砖流程节点

self.public_class.brick_process_num = 0  # 墙砖抓取次数

self.public_class.brick_process_result = None

self.public_class.brick_secondary_alignment_result = None

if self.brick_process_recorder is not None:

self.textEdit_log.append(

f"墙砖记录完成: {self.brick_process_recorder.session_dir}\n"

)

self.reset_rotation_history("brick")

# 判断是否是钢筋捆扎流程

elif self.public_class.rebar_process_state:

# 判断执行器放置区域

if self.public_class.rebar_process_node == "0-0":

# 获取机械臂控制器的数字输入信号

self.jaka.blinx_get_digital_input_status()

time.sleep(0.5)

# 将吸盘捆扎机的放置区的状态提出

xp_state = self.robot_DI_1

kzj_state = self.robot_DI_2

# 判断吸盘与捆扎机是否都在

if xp_state == 1 and kzj_state == 1:

self.public_class.rebar_process_node = "1-0"   # 如果两个末端执行器都在

# 判断如果吸盘在捆扎机不在

elif xp_state == 1 and kzj_state != 1:

self.public_class.rebar_process_node = "3-0"   # 如果是捆扎机就直接开始下一步

# 判断如果捆扎机在，吸盘不在

elif xp_state != 1 and kzj_state == 1:

self.public_class.rebar_process_node = "2-0"   # 需先放置捆扎机，在获取夹爪

# 如果两个都不在

elif xp_state != 1 and kzj_state != 1:

print("报警，请将末端执行器归位")

self.public_class.rebar_process_state = False

# 控制机械臂旋转180度

elif self.public_class.rebar_process_node == "1-0":

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-1"

# 角度是否到达，如果到达控制J6旋转90都

elif self.public_class.rebar_process_node == "1-1":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制末端移动90度

data = [float(self.public_class.joint_pos[0]), float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]),

float(self.public_class.joint_pos[5]) - 90.00]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-2"

# 角度是否到达，如果到达控制机械臂到达捆扎机上方

elif self.public_class.rebar_process_node == "1-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂到达捆扎机上方

data = [float(self.public_class.bundle_actuator_loc[0]),

float(self.public_class.bundle_actuator_loc[1]),

float(self.public_class.bundle_actuator_loc[2]) + 30.00,

float(self.public_class.bundle_actuator_loc[3]),

float(self.public_class.bundle_actuator_loc[4]),

float(self.public_class.bundle_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-3"

# 判断坐标是否到达，如果到达先打开快换夹具，在控制机械臂到达捆扎机位置

elif self.public_class.rebar_process_node == "1-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达捆扎机位置

data = [float(self.public_class.bundle_actuator_loc[0]),

float(self.public_class.bundle_actuator_loc[1]),

float(self.public_class.bundle_actuator_loc[2]),

float(self.public_class.bundle_actuator_loc[3]),

float(self.public_class.bundle_actuator_loc[4]),

float(self.public_class.bundle_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-4"

# 判断坐标是否到达，如果到达控制快换夹具关，并控制机械臂上升

elif self.public_class.rebar_process_node == "1-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 快换夹具开

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 0)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 10.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-5"

# 判断坐标是否到达，如果到达控制Y轴移动到

elif self.public_class.rebar_process_node == "1-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出吸盘放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1] + 100.00),

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-6"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.rebar_process_node == "1-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-7"

# 角度是否到达，如果到达控制机械臂到达捆扎机过度位置

elif self.public_class.rebar_process_node == "1-7":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂捆扎机过度位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1] + 50.00),

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-8"

# 判断坐标是否到达，如果到达控制机械臂旋转180度

elif self.public_class.rebar_process_node == "1-8":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.joint_pos[0]) + 180.00,

float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]),

float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]),

float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "1-9"

# 角度是否到达，如果到达控制机械臂到达初始位置

elif self.public_class.rebar_process_node == "1-9":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (

-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (

-0.1 <= error_j6 <= 0.1):

data = [float(self.public_class.initial_angle[0]),

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]),

float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]),

float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-0"

# 控制走放置吸盘流程

# 控制机械臂旋转180度

elif self.public_class.rebar_process_node == "2-0":

data = [float(self.public_class.joint_pos[0]) - 180.00, float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-1"

# 角度是否到达，如果到达控制J6旋转90都

elif self.public_class.rebar_process_node == "2-1":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制末端移动90度

data = [float(self.public_class.joint_pos[0]), float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5]) + 90.00]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-2"

# 角度是否到达，如果到达控制机械臂待放置位置

elif self.public_class.rebar_process_node == "2-2":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂待放置位置

data = [float(self.public_class.sucker_actuator_loc[0]),

float(self.public_class.sucker_actuator_loc[1]) + 100.00,

float(self.public_class.sucker_actuator_loc[2]) + 10.00,

float(self.public_class.sucker_actuator_loc[3]),

float(self.public_class.sucker_actuator_loc[4]),

float(self.public_class.sucker_actuator_loc[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-3"

# 判断坐标是否到达，如果到达先控制机械臂到达吸盘放置区位置上一公分位置

elif self.public_class.rebar_process_node == "2-3":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]) - 100.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-4"

# 判断坐标是否到达，并控制机械臂下降，再释放快换夹具

elif self.public_class.rebar_process_node == "2-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂到达吸盘位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 10.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-5"

time.sleep(1)

# 快换夹具释放

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.5)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 判断坐标是否到达，如果到达控制z轴上升

elif self.public_class.rebar_process_node == "2-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制机械臂退出吸盘放置位置

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2] + 30.00),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-6"

# 判断坐标是否到达，如果到达控制机械臂到达回收位置

elif self.public_class.rebar_process_node == "2-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

data = [float(self.public_class.initial_angle[0]) - 180.00,

float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "2-7"

# 角度是否到达，如果到达控制机械臂回到初始角度,并控制滑轨到达拍照点位

elif self.public_class.rebar_process_node == "2-7":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

self.public_class.rebar_process_node = "1-1"

# 开启流程

# 控制机械臂达到拍照位置

elif self.public_class.rebar_process_node == "3-0":

if self.public_class.new_data == None:

# 控制机械臂达到钢筋捆扎的拍照位置

data = [float(self.public_class.identify_loc1[0]), float(self.public_class.identify_loc1[1]),

float(self.public_class.identify_loc1[2]), float(self.public_class.identify_loc1[3]),

float(self.public_class.identify_loc1[4]), float(self.public_class.identify_loc1[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-1"

else:

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 控制机械臂达到钢筋捆扎的拍照位置

data = [float(self.public_class.identify_loc1[0]), float(self.public_class.identify_loc1[1]),

float(self.public_class.identify_loc1[2]), float(self.public_class.identify_loc1[3]),

float(self.public_class.identify_loc1[4]), float(self.public_class.identify_loc1[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-1"

self.jaka.blinx_set_analog_output(6, 26, 500)  # 滑轨绝对速度

self.jaka.blinx_set_analog_output(6, 25, -873.00)  # 滑轨绝对位置

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 4, 1)

self.jaka.blinx_set_digital_output(6, 4, 0)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 识别图像，并将结果进行Y方向的排序

elif self.public_class.rebar_process_node == "3-1":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-873.00))

if error_ai <= 1:

time.sleep(1)

print("进行拍照识别")

# 获取图像数据

self.public_class.mech_2d_image, self.public_class.mech_depth_map, self.public_class.mech_point_cloud = self.mechCam.GrabImages()

# 将2D图像进行图像识别

img_rec, data_list = self.yolo_iamge.blinx_rebar_image_rec(self.public_class.mech_2d_image)

# 将图像显示在界面中

# 获取图像高度和宽度

heigt, width = img_rec.shape[:2]

# 显示图像到界面

pixmap = QtGui.QImage(img_rec, width, heigt, QtGui.QImage.Format_RGB888)

pixmap = QtGui.QPixmap.fromImage(pixmap)

self.Image_Show_1.setPixmap(pixmap)

self.Image_Show_1.setScaledContents(True)  # 图像自适应窗口大小

# 深度图像显示

# 获取图像高度和宽度

depth_8bit = cv2.normalize(self.public_class.mech_depth_map, None, 0, 255,

cv2.NORM_MINMAX, cv2.CV_8UC1)

depth_color = cv2.applyColorMap(depth_8bit, cv2.COLORMAP_JET)

depth_rgb = cv2.cvtColor(depth_color, cv2.COLOR_BGR2RGB)

heigt, width = depth_rgb.shape[:2]

# 显示图像到界面

pixmap = QtGui.QImage(depth_rgb, width, heigt, QtGui.QImage.Format_RGB888)

pixmap = QtGui.QPixmap.fromImage(pixmap)

self.Image_Show_2.setPixmap(pixmap)

self.Image_Show_2.setScaledContents(True)  # 图像自适应窗口大小

# 将数据进行Y方向的排序

self._set_display_image(self.Image_Show_1, img_rec, QtGui.QImage.Format_RGB888)

self._set_display_image(self.Image_Show_2, depth_rgb, QtGui.QImage.Format_RGB888)

sorted_data = sorted(data_list, key=lambda x: x[1], reverse=True)

print(sorted_data)

# 将识别数据保存到公共类中

self.public_class.rebar_data_list = sorted_data

self.public_class.rebar_process_node = "3-2"

# 控制旋转45度

elif self.public_class.rebar_process_node == "3-2":

# 控制末端移动45度

data = [float(self.public_class.joint_pos[0]), float(self.public_class.joint_pos[1]),

float(self.public_class.joint_pos[2]), float(self.public_class.joint_pos[3]),

float(self.public_class.joint_pos[4]), float(self.public_class.joint_pos[5]) - 45.00]

self.jaka.blinx_joint_move(0, data, 50, 50)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-3"

# 根据识别数据进行标定，并先移动XY轴

elif self.public_class.rebar_process_node == "3-3" or self.public_class.rebar_process_node == "3-3-1":

if self.public_class.rebar_process_node == "3-3":

error_j1 = (float(self.public_class.new_data[0]) - float(

self.public_class.joint_pos[0]))

error_j2 = (float(self.public_class.new_data[1]) - float(

self.public_class.joint_pos[1]))

error_j3 = (float(self.public_class.new_data[2]) - float(

self.public_class.joint_pos[2]))

error_j4 = (float(self.public_class.new_data[3]) - float(

self.public_class.joint_pos[3]))

error_j5 = (float(self.public_class.new_data[4]) - float(

self.public_class.joint_pos[4]))

error_j6 = (float(self.public_class.new_data[5]) - float(

self.public_class.joint_pos[5]))

if (-0.1 <= error_j1 <= 0.1) and (-0.1 <= error_j2 <= 0.1) and (-0.1 <= error_j3 <= 0.1) and (

-0.1 <= error_j4 <= 0.1) and (-0.1 <= error_j5 <= 0.1) and (-0.1 <= error_j6 <= 0.1):

# 进行标定转换

x = self.public_class.rebar_data_list[int(self.public_class.rebar_process_num)][0]

y = self.public_class.rebar_data_list[int(self.public_class.rebar_process_num)][1]

test_robot_xy = np.dot(self.m_ini1, [x, y, 1])  # 仿射逆变换，得到坐标（x,y)

x = test_robot_xy[0] - 15.00

y = test_robot_xy[1] - 10.00

# 根据标定结果移动xy轴

data = [float(x), float(y),

float(self.public_class.tcp_pos[2]), float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]), float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-4"

else:

# 进行标定转换

x = self.public_class.rebar_data_list[int(self.public_class.rebar_process_num)][0]

y = self.public_class.rebar_data_list[int(self.public_class.rebar_process_num)][1]

test_robot_xy = np.dot(self.m_ini1, [x, y, 1])  # 仿射逆变换，得到坐标（x,y)

x = test_robot_xy[0] - 15.00

y = test_robot_xy[1] - 10.00

# 根据标定结果移动xy轴

data = [float(x), float(y),

float(self.public_class.tcp_pos[2]), float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]), float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-4"

# 根据高度，下降Z轴

elif self.public_class.rebar_process_node == "3-4":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

if self.public_class.rebar_process_num > 0:

# 根据高度下降Z

data = [float(self.public_class.tcp_pos[0]), float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 136.00 + 36, float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]), float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-5"

else:

# 根据高度下降Z

data = [float(self.public_class.tcp_pos[0]), float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 144.00, float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]), float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-5"

# 控制机械臂往内移动

elif self.public_class.rebar_process_node == "3-5":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 根据高度下降Z

data = [float(self.public_class.tcp_pos[0]) + 10.00, float(self.public_class.tcp_pos[1]) + 10.00,

float(self.public_class.tcp_pos[2]), float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]), float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-6"

# 控制Z轴继续下降一公分

elif self.public_class.rebar_process_node == "3-6":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 根据高度下降Z

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) - 10.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-7"

# 打开捆扎机，进行捆扎，并控制z轴上升

elif self.public_class.rebar_process_node == "3-7":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 打开捆扎机

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 7, 1)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 等待两秒

time.sleep(2)

# 关闭捆扎机

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 7, 0)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制Z轴上升

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) + 10.00,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-8"

# 控制XY进行移动

elif self.public_class.rebar_process_node == "3-8":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制

data = [float(self.public_class.tcp_pos[0]) - 10.00,

float(self.public_class.tcp_pos[1]) - 10.00,

float(self.public_class.tcp_pos[2]),

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-9"

# 控制Z轴上升

elif self.public_class.rebar_process_node == "3-9":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

# 控制Z轴上升

data = [float(self.public_class.tcp_pos[0]),

float(self.public_class.tcp_pos[1]),

float(self.public_class.tcp_pos[2]) + 100.60,

float(self.public_class.tcp_pos[3]),

float(self.public_class.tcp_pos[4]),

float(self.public_class.tcp_pos[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = data

self.public_class.rebar_process_node = "3-10"

# 判断是否还需要进行捆扎

elif self.public_class.rebar_process_node == "3-10":

error_x = (float(self.public_class.new_data[0]) - float(

self.public_class.tcp_pos[0]))

error_y = (float(self.public_class.new_data[1]) - float(

self.public_class.tcp_pos[1]))

error_z = (float(self.public_class.new_data[2]) - float(

self.public_class.tcp_pos[2]))

if (-0.1 <= error_x <= 0.1) and (-0.1 <= error_y <= 0.1) and (-0.1 <= error_z <= 0.1):

if self.public_class.rebar_process_num >= len(self.public_class.rebar_data_list) - 1:

# 回到初始位置

data = [float(self.public_class.identify_loc1[0]), float(self.public_class.identify_loc1[1]),

float(self.public_class.identify_loc1[2]), float(self.public_class.identify_loc1[3]),

float(self.public_class.identify_loc1[4]), float(self.public_class.identify_loc1[5])]

self.jaka.blinx_moveL(data, 250, 5000, 0)

self.public_class.new_data = None

self.public_class.rebar_process_node = "0-0"

self.public_class.rebar_process_state = False

else:

# 回到流程继续捆扎

self.public_class.rebar_process_node = "3-3-1"

self.public_class.rebar_process_num = self.public_class.rebar_process_num + 1

# endregion

# region page3标定

def get_image_mech(self):

self.pixel_point = []

self.lineEdit_P_x_1.setText("")

self.lineEdit_P_y_1.setText("")

self.lineEdit_P_x_2.setText("")

self.lineEdit_P_y_2.setText("")

self.lineEdit_P_x_3.setText("")

self.lineEdit_P_y_3.setText("")

self.lineEdit_P_x_4.setText("")

self.lineEdit_P_y_4.setText("")

if self.public_class.mech_connected:

self.public_class.mech_2d_image, self.public_class.mech_depth_map, self.public_class.mech_point_cloud = self.mechCam.GrabImages()

if self.public_class.mech_2d_image is not None:

self.biaoding_image = self.public_class.mech_2d_image

# 处理并显示2D图像

self.process_and_show_biaoding_image()

else:

QMessageBox.warning(mainWindow, "Error", "Mech3D相机未连接", QMessageBox.Ok)

def process_and_show_biaoding_image(self):

# 清除旧场景

self.scene_biaoding.clear()

# 转换图像格式

height, width = self.biaoding_image.shape[:2]

bytes_per_line = 3 * width

q_img = QtGui.QImage(self.biaoding_image.data, width, height,

bytes_per_line, QtGui.QImage.Format_RGB888)

# 创建QPixmap并添加到场景

pixmap = QtGui.QPixmap.fromImage(q_img)

self.pixmap_item = self.scene_biaoding.addPixmap(pixmap)

self.pixmap_item.setTransformationMode(QtCore.Qt.SmoothTransformation)

# print("pixmap_item 创建成功:", self.pixmap_item)  # 调试输出

# 设置场景大小

self.scene_biaoding.setSceneRect(0, 0, width, height)

# 重置视图变换

self.graphics_view_biaoding.resetTransform()

# 新增：初始自适应视图大小

self.fit_biaoding_image_to_view()

# 新增方法：自适应2D图像到视图

def fit_biaoding_image_to_view(self):

if not self.scene_biaoding.items():

return

# 获取视图和场景的尺寸

view_rect = self.graphics_view_biaoding.viewport().rect()

scene_rect = self.scene_biaoding.sceneRect()

# 计算合适的缩放比例

x_ratio = view_rect.width() / scene_rect.width()

y_ratio = view_rect.height() / scene_rect.height()

ratio = min(x_ratio, y_ratio)

# 应用缩放

self.graphics_view_biaoding.resetTransform()

self.graphics_view_biaoding.scale(ratio, ratio)

# 确保图像居中

self.center_biaoding_image()

# 新增方法：居中2D图像

def center_biaoding_image(self):

if not self.scene_biaoding.items():

return

self.graphics_view_biaoding.setSceneRect(self.scene_biaoding.sceneRect())

self.graphics_view_biaoding.centerOn(self.scene_biaoding.items()[0])

# endregion

# region 主界面按钮

def btn_systermRun_click(self):

self.public_class.is_continue_show = False  # 连续模式关闭

self.tabWidget.setCurrentIndex(0)

self.btn_systermRun.setStyleSheet(

"image: url(:/background/实时监测.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;border: 4px double rgb(0, 255, 0);")

self.btn_camSet.setStyleSheet(

"image: url(:/background/相机.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;")

self.btn_calibrate.setStyleSheet(

"image: url(:/background/坐标标定.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;")

self.label_systermRun.setStyleSheet("color:rgb(0,255,0);border:none;")

self.label_camSet.setStyleSheet("color:rgb(0,0,0);border:none;")

self.label_calibrate.setStyleSheet("color:rgb(0,0,0);border:none;")

def btn_camSet_click(self):

self.tabWidget.setCurrentIndex(1)

self.btn_camSet.setStyleSheet(

"image: url(:/background/相机.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;border: 4px double rgb(0, 255, 0);")

self.btn_systermRun.setStyleSheet(

"image: url(:/background/实时监测.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;")

self.btn_calibrate.setStyleSheet(

"image: url(:/background/坐标标定.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;")

self.label_systermRun.setStyleSheet("color:rgb(0,0,0);border:none;")

self.label_camSet.setStyleSheet("color:rgb(0,255,0);border:none;")

self.label_calibrate.setStyleSheet("color:rgb(0,0,0);border:none;")

def btn_calibrate_click(self):

self.public_class.is_continue_show = False  # 连续模式关闭

self.tabWidget.setCurrentIndex(2)

self.btn_systermRun.setStyleSheet(

"image: url(:/background/实时监测.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;")

self.btn_calibrate.setStyleSheet(

"image: url(:/background/坐标标定.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;border: 4px double rgb(0, 255, 0);")

self.btn_camSet.setStyleSheet(

"image: url(:/background/相机.png);background-color: rgb(0, 0, 150);border-radius: 30px; border: 2px groove rgb(255, 85, 0);border-style: none;color: white;")

self.label_systermRun.setStyleSheet("color:rgb(0,0,0);border:none;")

self.label_camSet.setStyleSheet("color:rgb(0,0,0);border:none;")

self.label_calibrate.setStyleSheet("color:rgb(0,255,0);border:none;")

# endregion

# region 流程按钮事件

# 吸盘取

def blinx_btn_suction_get(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.sucker_process = "0-0"

self.public_class.sucker_type = 0  # 0取  1放

self.public_class.sucker_state = True

else:

QMessageBox.warning(mainWindow, "Error", "请将滑轨回到初始位置", QMessageBox.Ok)

# 吸盘放

def blinx_btn_suction_set(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.sucker_process = "0-0"

self.public_class.sucker_type = 1  # 0取  1放

self.public_class.sucker_state = True

else:

QMessageBox.warning(mainWindow, "Error", "请将滑轨回到初始位置", QMessageBox.Ok)

# 捆扎机取

def blinx_btn_bundle_get(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.bundle_process = "0-0"

self.public_class.bundle_type = 0  # 0取   1放

self.public_class.bundle_state = True

else:

QMessageBox.warning(mainWindow, "Error", "请将滑轨回到初始位置", QMessageBox.Ok)

# 捆扎机放

def blinx_btn_bundle_set(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.bundle_process = "0-0"

self.public_class.bundle_type = 1  # 0取   1放

self.public_class.bundle_state = True

else:

QMessageBox.warning(mainWindow, "Error", "请将滑轨回到初始位置", QMessageBox.Ok)

# 初始化按钮

def blinx_btn_initialization(self):

self.jaka.blinx_set_analog_output(6, 26, 100)  # 滑轨绝对速度

self.jaka.blinx_set_analog_output(6, 25, -991)  # 滑轨绝对位置

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 4, 1)

self.jaka.blinx_set_digital_output(6, 4, 0)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 控制机械臂回到初始位置

data = [float(self.public_class.initial_angle[0]), float(self.public_class.initial_angle[1]),

float(self.public_class.initial_angle[2]), float(self.public_class.initial_angle[3]),

float(self.public_class.initial_angle[4]), float(self.public_class.initial_angle[5])]

self.jaka.blinx_joint_move(0, data, 50, 50)

# 将按钮启用

self.btn_suction_get.setEnabled(True)

self.btn_suction_set.setEnabled(True)

self.btn_bundle_get.setEnabled(True)

self.btn_bundle_set.setEnabled(True)

self.btn_porcelain.setEnabled(True)

self.btn_brick.setEnabled(True)

self.btn_rebar.setEnabled(True)

# 瓷砖流程按钮

def blinx_btn_porcelain(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.ceramic_process_state = True  # 瓷砖流程状态

self.public_class.ceramic_process_node = "0-0"  # 瓷砖流程节点

self.public_class.ceramic_process_num = 0  # 瓷砖流程抓取次数

self.public_class.ceramic_process_result = None

self.reset_rotation_history("ceramic")

else:

QMessageBox.warning(mainWindow, "Error", "请先初始化，将设备回到初始位置", QMessageBox.Ok)

# 砌砖流程按钮

def blinx_btn_brick(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.brick_process_state = True  # 墙砖流程状态

self.public_class.brick_process_node = "0-0"  # 墙砖流程节点

self.public_class.brick_process_num = 0  # 墙砖抓取次数

self.public_class.brick_process_result = None

self.public_class.brick_secondary_alignment_result = None

self.reset_rotation_history("brick")

self._start_brick_process_recording_session()

else:

QMessageBox.warning(mainWindow, "Error", "请先初始化，将设备回到初始位置", QMessageBox.Ok)

# 钢筋捆扎流程按钮

def blinx_btn_rebar(self):

# 获取滑轨当前位置

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.5)

print(self.public_class.ai_value)

error_ai = abs(float(self.public_class.ai_value) - float(-991))

if error_ai <= 1:

self.public_class.new_data = None

self.public_class.rebar_process_state = True  # 钢筋捆扎流程状态

self.public_class.rebar_process_node = "0-0"  # 钢筋捆扎流程节点

self.public_class.rebar_process_num = 0  # 钢筋捆扎次数

else:

QMessageBox.warning(mainWindow, "Error", "请先初始化，将设备回到初始位置", QMessageBox.Ok)

# endregion

# region功能函数:标定

# ch:存图 | en:save image

def save_jpg_biaoding(self):

now_time = time.strftime('%Y-%m-%d-%H-%M-%S', time.localtime(time.time()))  # 获取系统时间

if self.biaoding_image is None:

strError = "Save jpg failed:"

QMessageBox.warning(mainWindow, "Error", strError, QMessageBox.Ok)

else:

image = cv2.cvtColor(self.biaoding_image, cv2.COLOR_BGR2RGB)

cv2.imwrite('pic/biaoding1/image_' + str(now_time) + '.jpg', image)  # 保存2D图片

print("Save image_biaoding success")

def getPixel(self, event):

if self.biaoding_image is not None:

# 1. 获取鼠标点击的视图坐标

view_pos = event.pos()

# 2. 转换为场景坐标

scene_pos = self.graphics_view_biaoding.mapToScene(view_pos)

# 3. 转换为图像项坐标（考虑缩放、旋转）

if hasattr(self, 'pixmap_item') and self.pixmap_item:

item_pos = self.pixmap_item.mapFromScene(scene_pos)

# 4. 获取逆变换矩阵（处理缩放、旋转）

transform = self.pixmap_item.transform().inverted()[0]

pixel_pos = transform.map(item_pos)

x = int(pixel_pos.x())

y = int(pixel_pos.y())

# 5. 确保坐标在图像范围内

width = self.pixmap_item.pixmap().width()

height = self.pixmap_item.pixmap().height()

pixel_x = max(0, min(x, width - 1))

pixel_y = max(0, min(y, height - 1))

print(f"真实像素坐标: ({pixel_x}, {pixel_y})")

if len(self.pixel_point) <= 3:

self.pixel_point.append((pixel_x, pixel_y))

print(self.pixel_point)

for i in range(len(self.pixel_point) - 1, len(self.pixel_point)):

# if len(self.pixel_point)<=4:

# image_paint=image

cv2.circle(self.biaoding_image, (self.pixel_point[i][0], self.pixel_point[i][1]), 5,

(255, 0, 0),

thickness=-1)

cv2.putText(self.biaoding_image, "%s,%s" % (

str(len(self.pixel_point)) + ":(" + str(self.pixel_point[i][0]), str(self.pixel_point[i][1])) + ")",

(self.pixel_point[i][0], self.pixel_point[i][1]), cv2.FONT_HERSHEY_PLAIN,

3, (255, 0, 0), thickness=4)

if len(self.pixel_point) == 1:

self.lineEdit_P_x_1.setText(str(self.pixel_point[0][0]))

self.lineEdit_P_y_1.setText(str(self.pixel_point[0][1]))

elif len(self.pixel_point) == 2:

self.lineEdit_P_x_2.setText(str(self.pixel_point[1][0]))

self.lineEdit_P_y_2.setText(str(self.pixel_point[1][1]))

elif len(self.pixel_point) == 3:

self.lineEdit_P_x_3.setText(str(self.pixel_point[2][0]))

self.lineEdit_P_y_3.setText(str(self.pixel_point[2][1]))

elif len(self.pixel_point) == 4:

self.lineEdit_P_x_4.setText(str(self.pixel_point[3][0]))

self.lineEdit_P_y_4.setText(str(self.pixel_point[3][1]))

else:

return

# 转换图像格式

height, width = self.biaoding_image.shape[:2]

bytes_per_line = 3 * width

q_img = QtGui.QImage(self.biaoding_image.data, width, height,

bytes_per_line, QtGui.QImage.Format_RGB888)

# 创建QPixmap并添加到场景

pixmap = QtGui.QPixmap.fromImage(q_img)

pixmap_item = self.scene_biaoding.addPixmap(pixmap)

pixmap_item.setTransformationMode(QtCore.Qt.SmoothTransformation)

def pixel_to_object(self):  # 开始标定

# 坐标转为数组

if self.lineEdit_P_x_1.text() != "" and self.lineEdit_P_y_1.text() != "" and self.lineEdit_P_x_2.text() != "" and self.lineEdit_P_y_2.text() != "" and self.lineEdit_P_x_3.text() != "" and self.lineEdit_P_y_3.text() != "" and self.lineEdit_R_x_1.text() != "" and self.lineEdit_R_y_1.text() != "" and self.lineEdit_R_x_2.text() != "" and self.lineEdit_R_y_2.text() != "" and self.lineEdit_R_x_3.text() != "" and self.lineEdit_R_y_3.text() != "":

pts1 = np.float32([[int(self.lineEdit_P_x_1.text()), int(self.lineEdit_P_y_1.text())],

[int(self.lineEdit_P_x_2.text()), int(self.lineEdit_P_y_2.text())],

[int(self.lineEdit_P_x_3.text()), int(self.lineEdit_P_y_3.text())]])

pts2 = np.float32([[float(self.lineEdit_R_x_1.text()), float(self.lineEdit_R_y_1.text())],

[float(self.lineEdit_R_x_2.text()), float(self.lineEdit_R_y_2.text())],

[float(self.lineEdit_R_x_3.text()), float(self.lineEdit_R_y_3.text())]])

self.M = cv2.getAffineTransform(pts1, pts2)  # 仿射变化，仿射矩阵为2*3

print("M:", self.M)

test_robot_xy = np.dot(self.M, [int(self.lineEdit_P_x_4.text()), int(self.lineEdit_P_y_4.text()),

1])  # 仿射逆变换，得到坐标（x,y)

px = float(test_robot_xy[0])

py = float(test_robot_xy[1])

self.label_Ero_x.setText(str(round(float(self.lineEdit_R_x_4.text()) - px, 2)))

self.label_Ero_y.setText(str(round(float(self.lineEdit_R_y_4.text()) - py, 2)))

QMessageBox.warning(mainWindow, "OK", "标定成功", QMessageBox.Ok)

self.biaoding_ok = True

else:

QMessageBox.warning(mainWindow, "Error", "输入坐标为空", QMessageBox.Ok)

self.biaoding_ok = False

return

def matrix_saveto_ini(self):  # 保存标定结果

if self.biaoding_ok:

# region 配置文件读取

config = configparser.ConfigParser()

# -read读取ini文件

config.read('Config/config.ini', encoding="utf-8")

config.set("RelationalMatrix", "m1", str(self.M))

fh = open('Config/config.ini', 'w')

config.write(fh)

fh.close()

QMessageBox.warning(mainWindow, "OK", "保存标定参数成功", QMessageBox.Ok)

else:

QMessageBox.warning(mainWindow, "Error", "没有获取到标定参数矩阵", QMessageBox.Ok)

def lab_R_X_1_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_x_1.setText(self.label_j1.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_Y_1_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_y_1.setText(self.label_j2.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_X_2_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_x_2.setText(self.label_j1.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_Y_2_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_y_2.setText(self.label_j2.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_X_3_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_x_3.setText(self.label_j1.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_Y_3_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_y_3.setText(self.label_j2.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_X_4_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_x_4.setText(self.label_j1.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def lab_R_Y_4_clicked(self, event):

if self.joint_or_xyz:

self.lineEdit_R_y_4.setText(self.label_j2.text())

else:

QMessageBox.warning(mainWindow, "Error", "请将机器人切换到坐标控制", QMessageBox.Ok)

def calibration(self, piexl_x, piexl_y):  # 坐标转换：像素坐标转为机器人坐标

robot_xy = np.dot(self.m_ini, [piexl_x, piexl_y, 1])  # 仿射逆变换，得到坐标（x,y)

rx = round(float(robot_xy[0]), 1)  # 保留一位小数

ry = round(float(robot_xy[1]), 1)

return rx, ry

def calibration_parm_ini(self):  # 读取配置文件标定参数转换成矩阵参数

self.m_ini = self.strMatrix_to_Matrix(self.public_class.M)

self.m_ini1 = self.strMatrix_to_Matrix(self.public_class.M1)

print("m_ini:", self.m_ini)

def strMatrix_to_Matrix(self, strM):  # 字符串转2*3矩阵

out = strM.replace('[', '').replace(']', '')  # 去掉中括号

str = out.replace('\n', ' ')

res = re.sub(' +', ' ', str)  # 去掉一个或多个空格

# print("out:", out)

dlist = res.strip(' ').split(' ')  # 转换成一个list

listresult = []

for i in range(0, len(dlist)):

listresult.append(float(dlist[i]))  # 将字符串list转为float型的list

# print("listresult:", listresult)

darr = np.array(listresult)  # 将list转换为array

# print("darr:", darr)

resultM = darr.reshape(2, 3)  # 将array转换为2维(2,3)的矩阵

# print("result:", resultM)

return resultM

# endregion

# region功能函数:机械臂示教

# region 机器人控制器

def btn_controller_power_on_click(self):

self.plc.controller_power_on()

def btn_controller_power_off_click(self):

if self.jaka.blinx_shutdown():

self.textEdit_log.append("关闭控制器成功!\n")

self.label_jaka.setText("机器人连接失败")

self.label_jaka.setStyleSheet("color:rgb(255,0,0);border:none;")

self.textEdit_log.append("Error：机器人连接失败\n")

else:

self.textEdit_log.append("Error:关闭控制器失败\n")

def btn_robot_power_on_click(self):

if self.jaka.blinx_power_on():

self.textEdit_log.append("开启机器人电源成功!\n")

else:

self.textEdit_log.append("Error:开启机器人电源失败\n")

def btn_robot_power_off_click(self):

if self.jaka.blinx_power_off():

self.textEdit_log.append("关闭机器人电源成功!\n")

else:

self.textEdit_log.append("Error:关闭机器人电源失败\n")

def btn_enable_robot_click(self):

if self.jaka.blinx_enable_robot():

self.textEdit_log.append("开启机器上使能成功!\n")

else:

self.textEdit_log.append("Error:开启机器人上使能失败\n")

def btn_disable_robot_click(self):

if self.jaka.blinx_disable_robot():

self.textEdit_log.append("机器人下使能成功!\n")

else:

self.textEdit_log.append("Error:机器人下使能失败\n")

def btn_home_click(self):

home_point = [-90, 68.096, 108.580, 93.324, -90, 45]

self.jaka.blinx_joint_move(0, home_point, self.jaka.speeds_joint[1], 100)

def btn_play_click(self):

play_point = [-90, 48.804, 115.965, 105.230, -90, 45]

self.jaka.blinx_joint_move(0, play_point, self.jaka.speeds_joint[1], 100)

# endregion

# region 机器人状态获取

def get_jaka_state(self):

try:

self.jaka.blinx_get_joint_pos()

self.jaka.blinx_get_tcp_pos()

if self.joint_or_xyz == False:

if len(self.public_class.joint_pos) > 0:

self.label_j1.setText(str(round(self.public_class.joint_pos[0], 3)))

self.label_j2.setText(str(round(self.public_class.joint_pos[1], 3)))

self.label_j3.setText(str(round(self.public_class.joint_pos[2], 3)))

self.label_j4.setText(str(round(self.public_class.joint_pos[3], 3)))

self.label_j5.setText(str(round(self.public_class.joint_pos[4], 3)))

self.label_j6.setText(str(round(self.public_class.joint_pos[5], 3)))

elif self.joint_or_xyz:

if len(self.public_class.tcp_pos) > 0:

self.label_j1.setText(str(round(self.public_class.tcp_pos[0], 3)))

self.label_j2.setText(str(round(self.public_class.tcp_pos[1], 3)))

self.label_j3.setText(str(round(self.public_class.tcp_pos[2], 3)))

self.label_j4.setText(str(round(self.public_class.tcp_pos[3], 3)))

self.label_j5.setText(str(round(self.public_class.tcp_pos[4], 3)))

self.label_j6.setText(str(round(self.public_class.tcp_pos[5], 3)))

self.jaka.blinx_get_digital_input_status()

if len(self.public_class.din_status) > 0:

self.robot_DI_1 = self.public_class.din_status[0]

self.robot_DI_2 = self.public_class.din_status[1]

self.robot_DI_3 = self.public_class.din_status[2]

self.robot_DI_4 = self.public_class.din_status[3]

self.robot_DI_5 = self.public_class.din_status[4]

# print(self.robot_DI_1,self.robot_DI_2,self.robot_DI_3,self.robot_DI_4,self.robot_DI_5)

except Exception as e:

print("Error:获取机器人状态失败:", e)

# endregion

# region  关节控制

def btn_j1_add_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j1.text()) + float(self.lineEdit_step.text())

if -360 <= degree <= 360:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 0, self.lineEdit_speed.text(), self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j1.text()) + float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [distance, float(self.label_j2.text()), float(self.label_j3.text()),

float(self.label_j4.text()), float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 0, self.lineEdit_speed.text(), self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j1_subtract_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j1.text()) - float(self.lineEdit_step.text())

if -360 <= degree <= 360:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 0, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j1.text()) - float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [distance, float(self.label_j2.text()), float(self.label_j3.text()),

float(self.label_j4.text()), float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 0, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j2_add_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j2.text()) + float(self.lineEdit_step.text())

if -125 <= degree <= 125:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 1, self.lineEdit_speed.text(), self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j2.text()) + float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), distance, float(self.label_j3.text()),

float(self.label_j4.text()), float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 1, self.lineEdit_speed.text(), self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j2_subtract_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j2.text()) - float(self.lineEdit_step.text())

if -125 <= degree <= 125:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 1, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j2.text()) - float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), distance, float(self.label_j3.text()),

float(self.label_j4.text()), float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 1, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j3_add_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j3.text()) + float(self.lineEdit_step.text())

if -130 <= degree <= 130:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 2, self.lineEdit_speed.text(), self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j3.text()) + float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), distance,

float(self.label_j4.text()), float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 2, self.lineEdit_speed.text(), self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j3_subtract_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j3.text()) - float(self.lineEdit_step.text())

if -130 <= degree <= 130:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 2, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j3.text()) - float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), distance,

float(self.label_j4.text()), float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 2, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j4_add_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j4.text()) + float(self.lineEdit_step.text())

if -360 <= degree <= 360:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 3, self.lineEdit_speed.text(), self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j4.text()) + float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), float(self.label_j3.text()),

distance, float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 3, self.lineEdit_speed.text(), self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j4_subtract_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j4.text()) - float(self.lineEdit_step.text())

if -360 <= degree <= 360:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 3, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j4.text()) - float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), float(self.label_j3.text()),

distance, float(self.label_j5.text()), float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 3, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j5_add_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j5.text()) + float(self.lineEdit_step.text())

if -120 <= degree <= 120:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 4, self.lineEdit_speed.text(), self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j5.text()) + float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), float(self.label_j3.text()),

float(self.label_j4.text()), distance, float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 4, self.lineEdit_speed.text(), self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j5_subtract_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j5.text()) - float(self.lineEdit_step.text())

if -120 <= degree <= 120:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 4, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j5.text()) - float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), float(self.label_j3.text()),

float(self.label_j4.text()), distance, float(self.label_j6.text())]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 4, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j6_add_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j6.text()) + float(self.lineEdit_step.text())

if -360 <= degree <= 360:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 5, self.lineEdit_speed.text(), self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j6.text()) + float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), float(self.label_j3.text()),

float(self.label_j4.text()), float(self.label_j5.text()), distance]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 5, self.lineEdit_speed.text(), self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_j6_subtract_click(self):

if self.joint_or_xyz == False:

degree = float(self.label_j6.text()) - float(self.lineEdit_step.text())

if -360 <= degree <= 360:

self.jaka.blinx_jog(2, self.jaka.COORD_JOINT, 5, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

if self.joint_or_xyz == True:

distance = float(self.label_j6.text()) - float(self.lineEdit_step.text())

ref_pos = [0, 0, 90, 0, 90, 180]

cartesian_pose = [float(self.label_j1.text()), float(self.label_j2.text()), float(self.label_j3.text()),

float(self.label_j4.text()), float(self.label_j5.text()), distance]

ret = self.jaka.blinx_kine_inverse(ref_pos, cartesian_pose)

if ret:

self.jaka.blinx_jog(2, self.jaka.COORD_BASE, 5, -float(self.lineEdit_speed.text()),

self.lineEdit_step.text())

else:

QMessageBox.warning(mainWindow, "Error", "坐标不可达", QMessageBox.Ok)

def btn_clip_open_click(self):

self.plc.clip_open()

value = self.plc.clip_open_state()

print("打开状态：", bool(value))

# 判断夹爪是否打开完成

def clip_open_isok(self):

while True:

time.sleep(0.1)

try:

if bool(self.plc.clip_open_state()):

print("夹爪打开成功")

break

else:

print("夹爪未打开")

time.sleep(0.3)

except Exception as e:

print("夹爪打开失败")

continue

# 判断夹爪是否关闭完成

def clip_close_isok(self):

while True:

time.sleep(0.1)

try:

if bool(self.plc.clip_close_state()):

print("夹爪关闭成功")

break

else:

print("夹爪未关闭")

time.sleep(0.3)

except Exception as e:

print("夹爪关闭失败")

continue

# 判断是否回原点完成

def servo_reset_isok(self):

while True:

time.sleep(0.1)

try:

if bool(self.plc.servo_reset_state()):

print("回原点完成")

break

else:

print("等待回原点")

time.sleep(0.3)

except Exception as e:

continue

# 判断滑轨移动是否完成

def servo_absolute_finish_isok(self):

while True:

time.sleep(0.1)

try:

if bool(self.plc.servo_absolute_finish_state()):

print("滑轨移动完成")

break

else:

print("滑轨移动中")

time.sleep(0.3)

except Exception as e:

continue

def btn_step_add_click(self):

if float(self.lineEdit_step.text()) <= 20:

step = str(float(self.lineEdit_step.text()) + 1)

self.lineEdit_step.setText(step)

def btn_step_subtract_click(self):

if float(self.lineEdit_step.text()) >= 0.1:

step = str(float(self.lineEdit_step.text()) - 1)

self.lineEdit_step.setText(step)

def btn_speed_min_click(self):

if self.joint_or_xyz == False:

self.lineEdit_speed.setText('90')

else:

self.lineEdit_speed.setText('100')

def btn_speed_max_click(self):

if self.joint_or_xyz:

self.lineEdit_speed.setText('1500')

else:

self.lineEdit_speed.setText('180')

# 扎丝机开/关

def btn_zsj_open_click(self):

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 7, 1)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

def btn_zsj_close_click(self):

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 7, 0)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 吸盘打开/关闭

def btn_xipan_open_click(self):

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 6, 1)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

def btn_xipan_close_click(self):

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 6, 0)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 快换打开/关闭

def btn_kh_open_click(self):

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 1)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

def btn_kh_close_click(self):

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 8, 0)

time.sleep(0.2)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 滑轨向左/向右

def btn_servor_left_click(self):

self.jaka.blinx_get_analog_input(6,25)

time.sleep(0.01)

print(self.public_class.ai_value)

value=round(float(self.public_class.ai_value),2)-10

self.jaka.blinx_set_analog_output(6, 26, 500)#滑轨绝对速度

self.jaka.blinx_set_analog_output(6,25,value)#滑轨绝对位置

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 4, 1)

self.jaka.blinx_set_digital_output(6, 4, 0)

self.jaka.blinx_set_digital_output(6, 5, 0)

def btn_servor_right_click(self):

self.jaka.blinx_get_analog_input(6, 25)

time.sleep(0.01)

print(self.public_class.ai_value)

value = round(float(self.public_class.ai_value), 2) + 10

self.jaka.blinx_set_analog_output(6, 26, 500)

self.jaka.blinx_set_analog_output(6, 25, value)

self.jaka.blinx_set_digital_output(6, 5, 1)

self.jaka.blinx_set_digital_output(6, 4, 1)

self.jaka.blinx_set_digital_output(6, 4, 0)

self.jaka.blinx_set_digital_output(6, 5, 0)

# 滑轨启动/停止/回原点

def btn_servor_run_click(self):

9

def btn_servor_reset_click(self):

10

def absolute_speed_changed(self):

print(self.lineEdit_absolute_speed.text())

def absolute_loaction_changed(self):

print(self.lineEdit_absolute_loaction.text())

# endregion

# region 角度控制按钮

def btn_joint_click(self):

self.joint_or_xyz = False

self.lab1.setText("关节1：")

self.lab2.setText("关节2：")

self.lab3.setText("关节3：")

self.lab4.setText("关节4：")

self.lab5.setText("关节5：")

self.lab6.setText("关节6：")

self.btn_j1_add.setText("J1+")

self.btn_j1_subtract.setText("J1-")

self.btn_j2_add.setText("J2+")

self.btn_j2_subtract.setText("J2-")

self.btn_j3_add.setText("J3+")

self.btn_j3_subtract.setText("J3-")

self.btn_j4_add.setText("J4+")

self.btn_j4_subtract.setText("J4-")

self.btn_j5_add.setText("J5+")

self.btn_j5_subtract.setText("J5-")

self.btn_j6_add.setText("J6+")

self.btn_j6_subtract.setText("J6-")

self.btn_speed_min.setText("90")

self.btn_speed_max.setText("180")

self.lineEdit_speed.setText("90")

# endregion

# region 坐标控制按钮

def btn_xyz_click(self):

self.joint_or_xyz = True

self.lab1.setText("X  轴：")

self.lab2.setText("Y  轴：")

self.lab3.setText("Z  轴：")

self.lab4.setText("RX 轴：")

self.lab5.setText("RY 轴：")

self.lab6.setText("RZ 轴：")

self.btn_j1_add.setText("X+")

self.btn_j1_subtract.setText("X-")

self.btn_j2_add.setText("Y+")

self.btn_j2_subtract.setText("Y-")

self.btn_j3_add.setText("Z+")

self.btn_j3_subtract.setText("Z-")

self.btn_j4_add.setText("RX+")

self.btn_j4_subtract.setText("RX-")

self.btn_j5_add.setText("RY+")

self.btn_j5_subtract.setText("RY-")

self.btn_j6_add.setText("RZ+")

self.btn_j6_subtract.setText("RZ-")

self.btn_speed_min.setText("100")

self.btn_speed_max.setText("1500")

self.lineEdit_speed.setText("800")

# endregion

# region 判断字符串是否是数值类型

def is_number(self, s):

try:

float(s)

return True

except Exception as e:

print("数值转换整型错误:", e)

pass

try:

import unicodedata

unicodedata.numeric(s)

except Exception as e:

print("数值转换整型错误:", e)

return False

# endregion

# region退出程序，关闭线程

# region 关闭按钮

def closeEvent(self, event):

reply = QMessageBox.question(self, '本程序', "是否要退出程序？", QMessageBox.Yes | QMessageBox.No,

QMessageBox.No)

if reply == QtWidgets.QMessageBox.Yes:

if self.public_class.mech_connected:

self.mechCam.DisConnectCamera()

pid = os.getpid()

# SIGTERM表示终止进程

os.kill(pid, signal.SIGTERM)

event.accept()

else:

event.ignore()

# endregion

def _async_raise(self, tid, exctype):

"""raises the exception, performs cleanup if needed"""

tid = ctypes.c_long(tid)

if not inspect.isclass(exctype):

exctype = type(exctype)

res = ctypes.pythonapi.PyThreadState_SetAsyncExc(tid, ctypes.py_object(exctype))

if res == 0:

raise ValueError("invalid thread id")

elif res != 1:

# """if it returns a number greater than one, you're in trouble,

# and you should call it again with exc=NULL to revert the effect"""

ctypes.pythonapi.PyThreadState_SetAsyncExc(tid, None)

raise SystemError("PyThreadState_SetAsyncExc failed")

def stop_thread(self, thread):

self._async_raise(thread.ident, SystemExit)

# endregion

# region 主程序运行

if __name__ == "__main__":

app = QApplication(sys.argv)

mainWindow = QMainWindow()

blinx_xbmd_robot_vision = Blinx_XXXY_Robot_Vision()

blinx_xbmd_robot_vision.show()

sys.exit(app.exec_())

# endregion